# BYU Sewer Analysis

# Project ID:  CEEn_CPST_015

by

SNAC

Sophie Dahl

Clay Fullmer

Andy Hart

Natalie Gray

A Capstone Project 30% Completed Report

Submitted to

Mark Christensen

J-U-B

Department of Civil and Environmental Engineering

Brigham Young University

November 30, 2020

# Executive Summary

# PROJECT TITLE: 		BYU Sewer Analysis

# PROJECT ID: 		CEEn_CPST_015

# PROJECT SPONSOR: 	J-U-B

# TEAM NAME: 		SNAC

This project requires SNAC to create a model of the existing BYU sanitary sewer system and perform an analysis of the current performance of said sewer system and determine what locations are close to reaching capacity. The model will be created in a sew analysis software “Storm Water Management Model” (SWMM). Multiple tasks will be performed to gather the proper data by researching how BYU uses their land and convert the data into water flow. Other tasks will include identifying valleys in the sewer system and locating where it is close to capacity, determining the variance of flows during the day, and create the model using collected data.

This model is expected to be completed at the latest by the end of February 2021, and the timeline is as follows: Gathering land use, meter, and event data by the end of November. Start creating model and identify different valleys in BYU sewer system by the end of December 2020. The model should be finished by the end of January and we will begin to tun analysis and results by the end of February.

Table of Contents

Introduction......................................................................................................................1

Schedule...........................................................................................................................2

Assumptions and Limitations...........................................................................................4

Design, Analysis, and Results..........................................................................................5

Lessons Learned...............................................................................................................6

Conclusions......................................................................................................................7

Recommendations............................................................................................................8

Appendix..........................................................................................................................9

List of Figures

Figure 1 BYU Sewer System AutoCAD...........................................................................5

List of Tables

Table 1 SNAC Schedule.................................................................................................2

Table 2 Building Use Data.............................................................................Appendix B

Introduction

The BYU Sewer Analysis is a project analyzing the current capacity and usage of the sewers used by Brigham Young University. A model of the sewer system will be used to analyze the sewer system and locate areas that are close to capacity, especially during peak flow times. The model will be created by using meter data collected from BYU campus, research done by SNAC team members, and the information recorded in an excel file to use in the model. The model will be calibrated using the data collected from the meters on BYU campus, and the results will be analyzed to determine locations of concern.

The project is scheduled to be completed by the end of February 2021. The collection of data to use in the model is to be completed by the middle of December. The collected data will consist of peak water usage during sporting events, average water use for different types of land use, collecting the current meter data from around BYU campus, and creating an excel spreadsheet to store the collected data and calculate needed flows. Creating the model is scheduled to be completed by the middle of January 2021. Creating the model will involve using maps and other resources to determine where pipes are located on campus and represent them in the model. Once the model is completed and calibrated using meter data, a full analysis will be run to determine where the sewer system is close to capacity.

There are a few assumptions being made for creating the model:

- The pipes are assumed to be in an open flow system

- Assume that if a building is used for a specific land use it has the same average flow

- For peak hour events, we will assume that the capacity for the building used will be completely used

It is expected that the results of the project include a functional model of the existing BYU sewer system, a report describing the conditions modeled, including land use and related flow criteria, hydraulic criteria, inflow/infiltration assumptions, and estimated hourly and seasonal variations in flow; assumptions made relative to special events on campus, GIS maps graphically depicting percent of full pipe capacity under different governing conditions.

<Click to provide an overview of the project, tasks, schedule, assumptions, expectations, and requirements>

Schedule

Table 1. SNAC Schedule

| Week of | All | Clay | Sophie | Andy | Natalie |
| --- | --- | --- | --- | --- | --- |
| 10/26/2020 | Group Meeting | Hydraulic modeling | Hydraulic modeling | Hydraulic modeling | Hydraulic modeling |
| 11/2/2020 | Group Meeting Client Meeting | Hydraulic modeling | Land Management | Spreadsheet | Spreadsheet |
| 11/9/2020 | Group Meeting | Land Management | Land Management | Land Management | Land Management |
| 11/16/2020 | Group Meeting Client Meeting | Land Management | Contact BYU for event information | Land Management | Land Management |
| 11/23/2020 | Group Meeting | Land Management | Land Management | Land Management | Land Management |
| 11/30/2020 | Group Meeting Client Meeting | Introduction to modeling software | Introduction to modeling software | Introduction to modeling software | Introduction to modeling software |
| 12/7/2020 | Group Meeting | Model Building | Model Building | Model Building | Model Building |
| 12/14/2020 | Group Meeting Client Meeting | Model Building | Model Building | Model Building | Model Building |
| 12/21/2020 | Group Meeting | Model Building | Model Building | Model Building | Model Building |
| 12/28/2020 | Group Meeting Client Meeting | Model Building | Model Building | Model Building | Model Building |
| 1/4/2021 | Group Meeting | Model Building | Model Building | Model Building | Model Building |
| 1/11/2021 | Group Meeting Client Meeting | Model Building | Model Building | Model Building | Model Building |
| 1/18/2021 | Group Meeting | Calibrate Model | Calibrate Model | Update spreadsheet | Update spreadsheet |
| 1/25/2021 | Group Meeting Client Meeting | Calibrate Model | Calibrate Model | Update spreadsheet | Update spreadsheet |
| 2/1/2021 | Group Meeting | Analysis | Analysis | Analysis | Analysis |
| 2/8/2021 | Group Meeting Client Meeting | Create presentation | Create Presentation | Create Presentation | Create Presentation |
| 2/15/2021 | Group Meeting | Create Presentation | Create Presentation | Create Presentation | Create Presentation |
| 2/22/2021 | Group Meeting Client Meeting | Extra time | Extra time | Extra time | Extra time |
| 3/1/2021 | Group Meeting | Presentation | Presentation | Presentation | Presentation |

<Click to provide time line with important milestones, accomplishments, challenges, etc.>

Assumptions & Limitations

All aspects of the BYU sewer system are not known. Logical assumptions have been made to complete the project based on predictable factors that affect flow. Land use data flows are produced by using average data from research and are based upon square footage. The model will be based upon average flows and calibrated to peak flows. Pipe frictions and large on campus event sizes will also be estimated from research and flow data.

Limitations include the inability to work in person, the use of unfamiliar software for analysis, time, and inexperience. Our group will not work in person, so online communication will be vital. SWIMM is new to our group, but planned times to work on the software and internet helps should be sufficient to complete the analysis. The team will have about three months to finish the project from the date of this report. A professor specializing in water resources will advise team SNAC on demand.

<Click to provide concise explanations of the assumptions used in the projects and their limitations>

Design, Analysis & Results

The sewer design is in the preliminary stages. We are currently gathering data and becoming familiar with the EPA SWIMM software. Data about BYU campus is mainly shown in CAD files as shown in figure 1 below. Analysis and results will be reserved until next semester as stated in the introduction. The majority of time spent on the project will be from building the sewer system in the model. Results are only preliminary and are from practice with the software.

Figure 1. BYU sewer system, autoCAD

<Click to provide detailed descriptions of the design and analysis process as well as discussions of the results.  Use as much graphics (i.e. pictures, tables, figures etc.) with concise descrptions as possible to convey your message instead of paragraphs of words>

Lessons Learned

Team meetings have become an issue. Midterms and homework have taken over valuable meeting times. Team SNAC is committing the second hour of capstone every week to meet and discuss the project. With a new time and commitment from each team member, the project should continue quickly to completion.

<Click to list challenges encountered during the course of the project and how your team overcome it (see your status reports).  This is for others to learn and avoid in the future.  This is what defines you and your team among potential employers.  They value this morning than most other things in assessing a potential employee.>

Conclusions

Team SNAC has yet to come to any conclusions as we have not completed the project but are rater in the intermediate stages. At the conclusion of this project, we plan on being able to conclude how well the BYU sewer systems are able to handle the flow of water they are subjected to, as well as where they have ample room for growth.

<Click to discuss what your team has learned from the results of this project:  Explain what, how and why your team has arrived at the conclusions listed here (i.e. justifications and backup information in support of your conclusions)>

Recommendations

Our sponsor should continue should continue to stay in close contact with team SNAC. Questions about pipe data, especially elevations in AutoCAD, should be discussed.

<Click to list what your team recommends your sponsor to do (i.e. Perform follow-on analysis; assess, evaluate and propose to their customer for implementations and support etc.>

Appendix A

# Andrew Hart

801-616-0703 | Andrewhart521@gmail.com | linkedin.com/in/andrew-hart-37806a72/

Education

B.S. Civil Engineering | Brigham Young UniversityApril 2021

Minor in Ballroom Dance

GPA: 3.91

Academic Scholarship recipient from Fall 2017-Winter 2021

Experience

Structural Intern

Vector Engineering | August 2020—Present

- Analyze 4+ buildings daily to determine structural integrity for solar installation

- Communicate with 5+ clients a day to complete projects

- Prepare company plans and letters to be stamped and delivered to clients

Mechanics of Materials Teaching Assistant

Brigham Young University | September 2018—April 2020

- Organized a Mechanics of Materials class of 150+ students

- Created 10+ solution guides to aid students in solving engineering problems

- Taught students how to solve problems using engineering principles

Student Manager

Culinary Support Center, Brigham Young University | April 2018—April 2020

- Communicated between 4+ organizations to resolve delivery related problems

- Organized a team of drivers to fulfill 6+ deliveries in 4 hours

- Managed time effectively to complete deliveries on time

Skills

Excel

AutoCAD

Revit

Fast at learning new software

Management

Clay J. Fullmer

(435) 616-2568 | clayfullmer@gmail.com

Education

BS, Civil and Environmental Engineering                                               August 2016 – Present

Brigham Young University, Provo, Utah

- GPA: 3.3

- Class: Senior

- Expected Graduation: April 2021

- President, BYU American Society of Civil Engineers                                 2019 – Present

Professional Experience

Geotechnical Engineer, Intern                                                                      April 2020 – Present

Internmountain Geotechnical Environmental Services inc., Draper

- Performed over 500 excavation observations and in-field soil classifications for rural developments

- Authored an equivalent number of client reports under a professional engineer

- Procured soil samples and managed in-field geotechnical investigations

Liquefaction Research Assistant                                                     December 2018 – April 2020

Brigham Young University, Provo

- Created an expansive liquefaction digital database for Chile, New Zealand, and Japan

- Digitized over 200 peer reviewed papers for the Next Generation Liquefaction Project

- Created, modified, and reviewed Excel spreadsheets daily

Carpenter’s Apprentice                                                                                     June 2016 – 2018

Brigham Young University, Provo

- Participated in demolition work and remodels involving over 200 hours of framing

- Conducted concrete scanning and core-drilling under carpenters, civil engineers, and architects

- Built and assembled custom furniture for BYU clients

Activities

- Member, Earthquake Engineering Research Institute                                   2018 – Present

- Member, national qualifying BYU Concrete Canoe team                                2018 – 2019

- Brigham Young University Men’s Chorus                                                       2016 – 2017

Skills

- Hydraulics

- AutoCAD

- Revit

- ArcGIS

- Microsoft Office

Appendix B

Table 2. Building Use Data

| Bldg Abv | Current? | Name | Gross SqFt | Year Occupied | Major Use | Asset Class | Asset Type |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A12A | YES | ASPEN GROVE (ELK POINT 12A - LRG A-FRAME | 376 | 1963 | NON-INST | REVENUE | Building |
| A12B | YES | ASPEN GROVE (CABIN 12B - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| A51 | YES | AREA 51 | 9683 | 2016 | ADMIN | APPROPRIATED | Building |
| AA07 | YES | ASPEN GROVE (CABIN 07 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA08 | YES | ASPEN GROVE (CABIN 08 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA10 | YES | ASPEN GROVE (CABIN 10 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA11 | YES | ASPEN GROVE (CABIN 11 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA17 | YES | ASPEN GROVE (CABIN 17 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA18 | YES | ASPEN GROVE (CABIN 18 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA19 | YES | ASPEN GROVE (CABIN 19 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA21 | YES | ASPEN GROVE (CABIN 21 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA22 | YES | ASPEN GROVE (CABIN 22 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA24 | YES | ASPEN GROVE (CABIN 24 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA26 | YES | ASPEN GROVE (CABIN 26 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA27 | YES | ASPEN GROVE (CABIN 27 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA28 | YES | ASPEN GROVE (CABIN 28 - LARGE A-FRAME) | 376 | 1963 | NON-INST | REVENUE | Building |
| AA33 | YES | ASPEN GROVE (CABIN 33 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AA34 | YES | ASPEN GROVE (CABIN 34 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AA35 | YES | ASPEN GROVE (CABIN 35 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AA38 | YES | ASPEN GROVE (CABIN 38 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AA39 | YES | ASPEN GROVE (CABIN 39 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AA40 | YES | ASPEN GROVE (CABIN 40 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AA41 | YES | ASPEN GROVE (CABIN 41 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AA42 | YES | ASPEN GROVE (CABIN 42 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AA43 | YES | ASPEN GROVE (CABIN 43 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AA45 | YES | ASPEN GROVE (CABIN 45 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AA46 | YES | ASPEN GROVE (CABIN 46 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AA47 | YES | ASPEN GROVE (CABIN 47 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AA48 | YES | ASPEN GROVE (CABIN 48 - LARGE A-FRAME) | 376 | 1966 | NON-INST | REVENUE | Building |
| AACN | YES | ASPEN GROVE (ARTS & CRAFTS - NORTH) | 1841 | 1993 | NON-INST | REVENUE | Building |
| AACP | YES | ASPEN GROVE (ARTS AND CRAFTS PAVILION) |  |  | NON-INST | REVENUE | Facility |
| AACS | YES | ASPEN GROVE (ARTS & CRAFTS - SOUTH) | 715 | 1983 | NON-INST | REVENUE | Building |
| AALP | YES | ASPEN GROVE (ALPINE CABIN - MILLER PROPE | 1292 | 2013 | NON-INST | REVENUE | Building |
| AARR | YES | ASPEN GROVE (ARCHERY/RIFLE RANGE) |  |  | NON-INST | REVENUE | Facility |
| AASP | YES | ASPEN GROVE (ASPEN LODGE) | 23642 | 2000 | NON-INST | REVENUE | Building |
| ABAD | YES | ASPEN GROVE (BADMINTON) |  |  | NON-INST | REVENUE | Facility |
| ABFD | YES | ASPEN GROVE (BALL FIELD) |  |  | NON-INST | REVENUE | Facility |
| ABKB | YES | ASPEN GROVE (OUTDOOR BASKETBALL COURT) |  |  | NON-INST | REVENUE | Facility |
| ABLG | YES | ASPEN GROVE (RAYMOND E. BECKHAM LODGE) | 41933 | 2008 | NON-INST | REVENUE | Building |
| ABLP | YES | ASPEN GROVE (BECKHAM LODGE PARKING) |  | 2008 | NON-INST | REVENUE | Facility |
| ABPV | YES | ASPEN GROVE (BALL FIELD PAVILION) | 1800 | 1999 | NON-INST | REVENUE | Facility |
| ABRD | YES | ASPEN GROVE (ALUMNI BOARD CABIN) | 1372 | 1990 | NON-INST | REVENUE | Building |
| ABST | YES | ASPEN GROVE (BIKE STORAGE) |  |  | NON-INST | REVENUE | Facility |
| ACOW | YES | ASPEN GROVE (COWBOY VILLAGE) |  |  | NON-INST | REVENUE | Facility |
| ACRC | YES | ASPEN GROVE (CANYON RIM C - STAFF) | 512 | 1963 | NON-INST | REVENUE | Building |
| ACRD | YES | ASPEN GROVE (SMALL A-FRAME PAVILION) | 196 | 1963 | NON-INST | REVENUE | Facility |
| ACRM | YES | ASPEN GROVE (CENTRAL RESTROOM) | 550 | 1963 | NON-INST | REVENUE | Building |
| ACRR | YES | ASPEN GROVE (ARTS & CRAFTS AREA RESTROOM | 105 | 2009 | NON-INST | REVENUE | Building |
| ACRS | YES | ASPEN GROVE (CANYON RIM SHOWERS/LAUNDRY) | 850 | 1963 | NON-INST | REVENUE | Building |
| AEDH | YES | ASPEN GROVE (EMERALD DINING HALL) | 9077 | 1976 | NON-INST | REVENUE | Building |
| AEPS | YES | ASPEN GROVE (ELK POINT RESTROOM SHOWERS/ | 890 | 1963 | NON-INST | REVENUE | Building |
| AFLA | YES | ASPEN GROVE (FAMILY LODGE A) | 2310 | 1994 | NON-INST | REVENUE | Building |
| AFLB | YES | ASPEN GROVE (FAMILY LODGE B) | 2310 | 1994 | NON-INST | REVENUE | Building |
| AFLC | YES | ASPEN GROVE (FAMILY LODGE C) | 2310 | 1994 | NON-INST | REVENUE | Building |
| AFLD | YES | ASPEN GROVE (FAMILY LODGE D) | 2310 | 1994 | NON-INST | REVENUE | Building |
| AGGW | YES | AGRONOMY WATERING |  |  |  | BILLING | Meter |
| AGLF | YES | ASPEN GROVE (MINIATURE GOLF) |  |  | NON-INST | REVENUE | Facility |
| AGPL | YES | ASPEN GROVE (SWIMMING POOL) |  |  | NON-INST | REVENUE | Facility |
| AGRC | YES | ASPEN GROVE (ROPES COURSE) |  |  | NON-INST | REVENUE | Facility |
| AHAC | YES | ASPEN GROVE (HUISH ACTIVITY CENTER) | 5120 | 1991 | NON-INST | REVENUE | Building |
| AHC1 | YES | ASPEN GROVE (HARRIS CABIN I) | 720 | 1925 | NON-INST | REVENUE | Building |
| AHC2 | YES | ASPEN GROVE (HARRIS CABIN II) | 390 | 1925 | NON-INST | REVENUE | Building |
| AHSP | YES | ASPEN GROVE (HORSE SHOE PITS) |  |  | NON-INST | REVENUE | Facility |
| ANDH | NO | ANDERSON HOUSE | 1981 | 2019 | ADMIN | APPROPRIATED | Discontinued |
| AOVR | YES | ASPEN GROVE (OVERLOOK CABIN) | 2510 | 2008 | NON-INST | REVENUE | Building |
| APHS | YES | ASPEN GROVE (PLAYHOUSE AREA) |  |  | NON-INST | REVENUE | Facility |
| APLG | YES | ASPEN GROVE (PLAYGROUND) |  |  | NON-INST | REVENUE | Facility |
| APMP | YES | ASPEN GROVE (PUMP HOUSE-OWNED BY NFSSD) | 470 | 1963 | NON-INST | REVENUE | Building |
| APPV | YES | ASPEN GROVE (SWIMMING POOL PAVILION) |  |  | NON-INST | REVENUE | Facility |
| APRG | YES | ASPEN GROVE (PROGRAM CLUB HOUSE) | 500 | 1995 | NON-INST | REVENUE | Building |
| APRK | YES | ASPEN GROVE (COVERED PARKING) |  |  | NON-INST | REVENUE | Facility |
| APSP | YES | ASPEN GROVE (PIRATE SHIP AND PLAY AREA) |  |  | NON-INST | REVENUE | Facility |
| APTB | YES | ASPEN GROVE (PAINTBALL AREA) |  |  | NON-INST | REVENUE | Facility |
| AR01 | YES | ASPEN GROVE (CABIN 01 - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| AR02 | YES | ASPEN GROVE (CABIN 02 - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| AR03 | YES | ASPEN GROVE (CABIN 03 - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| AR04 | YES | ASPEN GROVE (CABIN 04 - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| AR05 | YES | ASPEN GROVE (CABIN 05 - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| AR06 | YES | ASPEN GROVE (CABIN 06 - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| AR09 | YES | ASPEN GROVE (CABIN 09 - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| AR13 | YES | ASPEN GROVE (CABIN 13 - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| AR14 | YES | ASPEN GROVE (CABIN 14 - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| AR15 | YES | ASPEN GROVE (CABIN 15 - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| AR16 | YES | ASPEN GROVE (CABIN 16 - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| AR20 | YES | ASPEN GROVE (CABIN 20 - RECTANGULAR) | 327 | 1925 | NON-INST | REVENUE | Building |
| ARQT | YES | ASPEN GROVE (RACKETBALL COURTS) | 3270 | 1983 | NON-INST | REVENUE | Building |
| ASAT | YES | ASPEN GROVE (STAGE & AMPHITHEATRE) | 560 | 1989 | NON-INST | REVENUE | Building |
| ASB | YES | SMOOT (ABRAHAM O) ADMINISTRATION BUILDIN | 98123 | 1961 | ADMIN | APPROPRIATED | Building |
| ASHP | YES | ASPEN GROVE (SHOP) | 2967 | 1981 | NON-INST | REVENUE | Building |
| ASLD | YES | ASPEN GROVE (SLIDE) |  |  | NON-INST | REVENUE | Facility |
| ASNO | YES | ASPEN GROVE (SNOWSHOE LODGE) | 4799 | 1977 | NON-INST | REVENUE | Building |
| ASPN | YES | ASPEN GROVE - GENERAL |  |  | NON-INST | REVENUE | General Site |
| AST3 | YES | ASPEN GROVE (STAFF CABIN 3 - GIRLS WEST) | 200 | 1925 | NON-INST | REVENUE | Building |
| ASTA | YES | ASPEN GROVE (STORAGE A) | 377 | 1983 | NON-INST | REVENUE | Building |
| ASTF | YES | ASPEN GROVE (STAFF CABIN - GIRLS EAST) | 224 | 1925 | NON-INST | REVENUE | Building |
| ASTO | YES | ASPEN GROVE (STORAGE B,C,D) | 670 | 1983 | NON-INST | REVENUE | Building |
| ASUN | YES | ASPEN GROVE (SUNRISE CABIN) | 3039 | 2000 | NON-INST | REVENUE | Building |
| ASVB | YES | ASPEN GROVE (SAND VOLLEYBALL) |  |  | NON-INST | REVENUE | Facility |
| ASWM | YES | ASPEN GROVE (SWIMMING POOL COMPLEX) | 2219 | 1989 | NON-INST | REVENUE | Building |
| ATAT | YES | ASPEN GROVE (TEEN AMPHITHEATER) |  |  | NON-INST | REVENUE | Facility |
| ATCT | YES | ASPEN GROVE (TENNIS COURTS) |  |  | NON-INST | REVENUE | Facility |
| ATP1 | YES | ASPEN GROVE (SMALL A-FRAME PAVILION) | 196 | 1966 | NON-INST | REVENUE | Facility |
| ATP2 | YES | ASPEN GROVE (SMALL A-FRAME PAVILION) | 196 | 1966 | NON-INST | REVENUE | Facility |
| ATPS | YES | ASPEN GROVE (TALL PINES SHOWERS/LAUNDRY) | 678 | 1966 | NON-INST | REVENUE | Building |
| ATRP | YES | ASPEN GROVE (TRAMPOLINE) |  |  | NON-INST | REVENUE | Facility |
| AWPK | YES | ASPEN GROVE (WATER PARK) |  |  | NON-INST | REVENUE | Facility |
| AYRT | YES | ASPEN GROVE (YURT) |  |  | NON-INST | REVENUE | Facility |
| B01 | YES | B01 (GHOST TOWN FRONT/COOKHOUSE) | 454 | 1968 | ADMIN | APPROPRIATED | Building |
| B02 | YES | B02 (GHOST TOWN FRONT & THE ADVOCATE) | 320 | 1968 | ADMIN | APPROPRIATED | Building |
| B03 | YES | B03 (GHOST TOWN FRONT & BLACKSMITH STRG) | 245 | 1968 | ADMIN | APPROPRIATED | Building |
| B04 | YES | B04 (GHOST TOWN FRONT & DOOR STORAGE) | 793 | 1975 | ADMIN | APPROPRIATED | Building |
| B05 | YES | B05 (GHOST TOWN FRONT & GNRL MDSE STRG) | 406 | 1975 | ADMIN | APPROPRIATED | Building |
| B06 | YES | B06 (GHOST TOWN FRONT & WAGON WHL STRG) | 173 | 1975 | ADMIN | APPROPRIATED | Building |
| B07 | YES | B07 (GHOST TOWN FRONT & CRATE STORAGE) | 227 | 1975 | ADMIN | APPROPRIATED | Building |
| B44 | YES | B44 (MOTION PICTURE STUDIOS) - STORAGE | 19143 | 1967 | ADMIN | APPROPRIATED | Building |
| B45 | YES | B45 (GEOLOGY STORAGE) | 1932 | 1967 | ACADEMIC | APPROPRIATED | Building |
| B66 | YES | B66 (CERAMICS, SCULP, TECH LAB BLDG) | 21322 | 1976 | ACADEMIC | APPROPRIATED | Building |
| B67 | YES | B67 (OUTDOORS UNLIMITED/MUSEUM OF PEOPLE | 27885 | 1985 | ACADEMIC | APPROPRIATED | Building |
| B73 | YES | B73 (SERVICE BUILDING - PAINT) | 1036 | 1960 | ADMIN | APPROPRIATED | Building |
| BELL | YES | CENTENNIAL CARILLON TOWER | 1418 | 1975 | ACADEMIC | APPROPRIATED | Building |
| BLWH | YES | BARLOW CENTER FACULTY RESIDENCE | 2094 | 2011 | HOUSING | REVENUE | Building |
| BNSN | YES | BENSON (EZRA TAFT) BUILDING | 191325 | 1995 | ACADEMIC | APPROPRIATED | Building |
| BRLW | YES | BARLOW (MILTON A) CENTER | 10690 | 1999 | ACADEMIC | REVENUE | Building |
| BRMB | YES | BRIMHALL (GEORGE H) BUILDING | 40276 | 1919 | ACADEMIC | APPROPRIATED | Building |
| BRWB | YES | BREWSTER (SAM F) BUILDING | 86104 | 1962 | ADMIN | APPROPRIATED | Building |
| BTWR | YES | BAND TOWER | 60 | 2011 | ACADEMIC | APPROPRIATED | Facility |
| BUS1 | YES | BUS STOP CTB | 56 | 2015 | AUXILIARY | APPROPRIATED | Facility |
| BUS3 | YES | BUS STOP NORTH | 185 | 1989 | AUXILIARY | APPROPRIATED | Facility |
| BYUB | YES | BYU BROADCASTING | 101744 | 2010 | ACADEMIC | REVENUE | Building |
| BYUI | YES | Brigham Young University - Idaho |  |  | ACADEMIC | CHURCH | General Site |
| BYUW | YES | BYU WEST CAMPUS |  |  |  | Investment Property | General Site |
| CANC | YES | CANNON (GEORGE Q) CENTER (HELAMAN HALLS) | 47643 | 2008 | AUXILIARY | REVENUE | Building |
| CANP | YES | CANNON CENTER PATIO |  |  | HOUSING | REVENUE | Facility |
| CB | YES | CLYDE (W W) ENGINEERING BUILDING | 203575 | 1973 | ACADEMIC | APPROPRIATED | Building |
| CCPB | YES | CAMPUS CONCESSIONS PREPARATION BUILDING | 7738 | 1964 | AUXILIARY | REVENUE | Building |
| CFS1 | YES | CHILD & FAMILY STUDIES LAB PLAYGROUND ST | 121 | 2005 | ACADEMIC | APPROPRIATED | Building |
| CFS2 | YES | CHILD & FAMILY STUDIES LAB PLAYGROUND ST | 167 | 2005 | ACADEMIC | APPROPRIATED | Building |
| CGEN | YES | COGEN POWER |  |  |  | BILLING | Meter |
| CMB | YES | CHEMICALS MANAGEMENT BUILDING | 6870 | 1993 | ACADEMIC | APPROPRIATED | Building |
| CMPS | YES | GENERAL CAMPUS-APPROPRIATED |  |  |  | APPROPRIATED | General Site |
| COAL | YES | HEATING PLANT COAL AREA | 0 | 1950 | ADMIN | APPROPRIATED | Facility |
| CONE | YES | CREAMERY ON NINTH EAST | 12116 | 1999 | AUXILIARY | REVENUE | Building |
| CONF | YES | BYU CONFERENCE CENTER | 52033 | 1982 | ACADEMIC | APPROPRIATED | Building |
| CONP | YES | CONFERENCE CENTER EAST PATIO | 5832 | 2004 | ACADEMIC | APPROPRIATED | Facility |
| CSC | YES | CULINARY SUPPORT CENTER | 36099 | 1964 | AUXILIARY | REVENUE | Building |
| CTB | YES | CRABTREE (ROLAND A) TECHNOLOGY BUILDING | 99448 | 1985 | ACADEMIC | APPROPRIATED | Building |
| DI | YES | DESERET INDUSTRIES |  |  |  | CHURCH | Building |
| DIST | YES | DUMMY SCHEDULING - SCHOOL DISTRICTS | 0 | 1977 |  | SCHEDULING RECORD | Non-Asset |
| EB | YES | ENGINEERING BUILDING | 184343 | 2018 | ACADEMIC | APPROPRIATED | Building |
| ELLB | YES | ELLSWORTH (LEO) BUILDING | 74557 | 1980 | ACADEMIC | APPROPRIATED | Building |
| ELLE | YES | ELLSWORTH BUILDING COVERED PARKING - EAS |  | 2010 | ADMIN | APPROPRIATED | Facility |
| ELLW | YES | ELLSWORTH BUILDING COVERED PARKING - WES |  | 2010 | ADMIN | APPROPRIATED | Facility |
| ERL | YES | ENGINEERING RESEARCH LAB | 16331 | 2018 | ACADEMIC | APPROPRIATED | Building |
| ESC | YES | EYRING (CARL F) SCIENCE CENTER | 187590 | 1950 | ACADEMIC | APPROPRIATED | Building |
| FLSR | YES | FOREIGN LANGUAGE STUDENT RESIDENCE | 43980 | 1992 | ACADEMIC | REVENUE | Building |
| FPH | YES | FORMER PRESIDENTS' HOME | 5616 | 1925 | ACADEMIC | APPROPRIATED | Building |
| FSP | YES | FOREST SERVICE PROPERTIES | 0 |  | NON-INST | BILLING | Building |
| FSSS | YES | FILM STUDENT SUPPORT SERVICES | 4840 | 1982 | ADMIN | APPROPRIATED | Building |
| FTGB | YES | FOOD TO GO BUILDING | 3459 | 2013 | AUXILIARY | REVENUE | Building |
| GH | YES | UNIVERSITY GUEST HOUSE | 5498 | 2014 | ADMIN | REVENUE | Building |
| GITR | YES | SCHEDULING - GOLF IN THE ROUND | 0 |  | ACADEMIC | SCHEDULING RECORD | Non-Asset |
| GLFG | YES | GOLF GREEN |  |  | ACADEMIC | APPROPRIATED | Facility |
| GSTG | YES | GOLF STORAGE - SFH TENNIS COURTS | 200 | 1994 | ACADEMIC | APPROPRIATED | Building |
| HAWF | YES | HAWS FIELD |  | 1999 | ACADEMIC | APPROPRIATED | Facility |
| HBLL | YES | LEE (HAROLD B) LIBRARY | 715983 | 1961 | ACADEMIC | APPROPRIATED | Building |
| HC | YES | HINCKLEY (GORDON B) ALUMNI & VISITORS CE | 85691 | 2007 | ADMIN | DESIGNATED ACCOUNT | Building |
| HCEB | YES | HARMAN (CAROLINE HEMENWAY) CONTINUING ED | 142553 | 1982 | ACADEMIC | REVENUE | Building |
| HCGB | YES | HINCKLEY CENTER GAZEBO | 230 | 2007 | ADMIN | DESIGNATED ACCOUNT | Facility |
| HFAC | YES | HARRIS (FRANKLIN S) FINE ARTS CENTER | 292817 | 1964 | ACADEMIC | APPROPRIATED | Building |
| HGB | YES | GRANT (HEBER J) BUILDING | 26463 | 1925 | ACADEMIC | APPROPRIATED | Building |
| HHB1 | YES | HELAMAN HALLS BIKE PAVILION 1 |  |  | HOUSING | REVENUE | Facility |
| HHB2 | YES | HELAMAN HALLS BIKE PAVILION 2 |  |  | HOUSING | REVENUE | Facility |
| HHB3 | YES | HELAMAN HALLS BIKE PAVILION 3 |  |  | HOUSING | REVENUE | Facility |
| HHB4 | YES | HELAMAN HALLS BIKE PAVILION 4 |  |  | HOUSING | REVENUE | Facility |
| HHB5 | YES | HELAMAN HALLS BIKE PAVILION 5 |  |  | HOUSING | REVENUE | Facility |
| HHB6 | YES | HELAMAN HALLS BIKE PAVILION 6 |  |  | HOUSING | REVENUE | Facility |
| HHC1 | YES | HELAMAN HALLS COURT 1 |  |  | HOUSING | REVENUE | Facility |
| HHC2 | YES | HELAMAN HALLS COURT 2 |  |  | HOUSING | REVENUE | Facility |
| HHC3 | YES | HELAMAN HALLS COURT 3 |  |  | HOUSING | REVENUE | Facility |
| HHC4 | YES | HELAMAN HALLS COURT 4 |  |  | HOUSING | REVENUE | Facility |
| HHC5 | YES | HELAMAN HALLS COURT 5 |  |  | HOUSING | REVENUE | Facility |
| HL | YES | HELAMAN HALLS GENERAL |  |  |  | BILLING | General Site |
| HLB | YES | HELAMAN HALLS (B) HINCKLEY (IRA N) HALL | 48386 | 1958 | HOUSING | REVENUE | Building |
| HLBC | YES | HELAMAN HALLS BUILDING B CHILLER | 331 | 1958 | HOUSING | APPROPRIATED | Building |
| HLC | YES | HELAMAN HALLS (C) CHIPMAN (STEVEN L) HAL | 47170 | 1958 | HOUSING | REVENUE | Building |
| HLCC | YES | HELAMAN HALLS BUILDING C CHILLER | 365 | 1958 | HOUSING | APPROPRIATED | Building |
| HLD | YES | HELAMAN HALLS (D) JOHN (DAVID) HALL | 46519 | 1958 | HOUSING | REVENUE | Building |
| HLE | YES | HELAMAN HALLS (E) TAYLOR (THOMAS N) HALL | 46978 | 1958 | HOUSING | REVENUE | Building |
| HLEC | YES | HELAMAN HALLS BUILDING E CHILLER | 365 | 1958 | HOUSING | APPROPRIATED | Building |
| HLF | YES | HELAMAN HALLS (F) STOVER (WALTER F) HALL | 46977 | 1958 | HOUSING | REVENUE | Building |
| HLFC | YES | HELAMAN HALLS BUILDING F CHILLER | 362 | 1958 | HOUSING | APPROPRIATED | Building |
| HLG | YES | HELAMAN HALLS (G) BUDGE (WILLIAM) HALL | 48890 | 1959 | HOUSING | REVENUE | Building |
| HLGC | YES | HELAMAN HALLS BUILDING G CHILLER | 332 | 1959 | HOUSING | APPROPRIATED | Building |
| HLH | YES | HELAMAN HALLS (H) MERRILL (MARRINER WOOD | 46974 | 1959 | HOUSING | REVENUE | Building |
| HLHC | YES | HELAMAN HALLS BUILDING H CHILLER | 364 | 1959 | HOUSING | APPROPRIATED | Building |
| HLI | YES | HELAMAN HALLS (I) MAY (JEAN FOSSUM) HALL | 46976 | 1970 | HOUSING | REVENUE | Building |
| HLIC | YES | HELAMAN HALLS BUILDING I CHILLER | 364 | 1970 | HOUSING | APPROPRIATED | Building |
| HLJ | YES | HELAMAN HALLS (J) BUILDING 9 | 51054 | 2010 | HOUSING | REVENUE | Building |
| HLJC | YES | HELAMAN HALLS BUILDING J CHILLER | 509 | 2010 | HOUSING | APPROPRIATED | Building |
| HLPS | YES | HELAMAN HALLS PAVILION STORAGE (DINING S | 180 | 1975 | AUXILIARY | REVENUE | Building |
| HLPV | YES | HELAMAN HALLS PAVILION | 2658 | 1977 | HOUSING | REVENUE | Facility |
| HLRA | YES | HELAMAN HALLS RECREATION AREA (FIELDS) |  | 1958 | ACADEMIC | APPROPRIATED | Facility |
| HOSP | YES | SCHEDULING - HOSPITALS | 0 | 1971 |  | SCHEDULING RECORD | Non-Asset |
| HR | YES | HERITAGE HALLS - GENERAL |  |  |  | BILLING | General Site |
| HR02 | YES | HERITAGE HALLS (02) | 69964 | 2015 | HOUSING | REVENUE | Building |
| HR03 | YES | HERITAGE HALLS (03) | 82661 | 2013 | HOUSING | REVENUE | Building |
| HR04 | YES | HERITAGE HALLS (04) | 82678 | 2013 | HOUSING | REVENUE | Building |
| HR05 | YES | HERITAGE HALLS (05) | 69938 | 2015 | HOUSING | REVENUE | Building |
| HR06 | YES | HERITAGE HALLS (06) | 82700 | 2017 | HOUSING | REVENUE | Building |
| HR07 | YES | HERITAGE HALLS (07) | 91211 | 2015 | HOUSING | REVENUE | Building |
| HR08 | YES | HERITAGE HALLS (08) | 82623 | 2015 | HOUSING | REVENUE | Building |
| HR09 | YES | HERITAGE HALLS (09) | 69540 | 2011 | HOUSING | REVENUE | Building |
| HR10 | YES | HERITAGE HALLS (10) | 69561 | 2011 | HOUSING | REVENUE | Building |
| HR11 | YES | HERITAGE HALLS (11) | 69568 | 2011 | HOUSING | REVENUE | Building |
| HR12 | YES | HERITAGE HALLS (12) | 83420 | 2011 | HOUSING | REVENUE | Building |
| HR13 | YES | HERITAGE HALLS (13) | 0 | 2020 | HOUSING | REVENUE | Building |
| HR14 | YES | HERITAGE HALLS (14) | 0 | 2020 | HOUSING | REVENUE | Building |
| HR15 | YES | HERITAGE HALLS (15) | 79683 | 2013 | HOUSING | REVENUE | Building |
| HR16 | YES | HERITAGE HALLS (16) | 79497 | 2013 | HOUSING | REVENUE | Building |
| HR3P | NO | HERITAGE HALLS PHASE 3 METER |  |  |  | BILLING | Meter |
| HRB1 | YES | HERITAGE HALLS BIKE PAVILION 1 |  | 2011 | HOUSING | REVENUE | Facility |
| HRB2 | YES | HERITAGE HALLS BIKE PAVILION 2 |  | 2011 | HOUSING | REVENUE | Facility |
| HRB3 | YES | HERITAGE HALLS BIKE PAVILION 3 |  | 2013 | HOUSING | REVENUE | Facility |
| HRB4 | YES | HERITAGE HALLS BIKE PAVILION 4 |  | 2013 | HOUSING | REVENUE | Facility |
| HRB5 | YES | HERITAGE HALLS BIKE PAVILION 5 |  | 2013 | HOUSING | REVENUE | Facility |
| HRB6 | YES | HERITAGE HALLS BIKE PAVILION 6 |  | 2013 | HOUSING | REVENUE | Facility |
| HRB7 | YES | HERITAGE HALLS BIKE PAVILION 7 |  | 2015 | HOUSING | REVENUE | Facility |
| HRB8 | YES | HERITAGE HALLS BIKE PAVILION 8 |  | 2015 | HOUSING | REVENUE | Facility |
| HRC | YES | HERITAGE HALLS CHILLER | 1248 | 1964 | HOUSING | APPROPRIATED | Building |
| HRC1 | NO | HERITAGE HALLS COURT 1 |  | 2011 | HOUSING | REVENUE | Facility |
| HRCB | YES | CLARK (HERALD R) BUILDING | 30879 | 1952 | ACADEMIC | APPROPRIATED | Building |
| HRCN | YES | HERITAGE HALLS CENTRAL BUILDING | 35679 | 2017 | HOUSING | REVENUE | Building |
| INR1 | YES | INTRAMURAL FIELDS RESTROOM (IWRA) | 814 | 2010 | ACADEMIC | APPROPRIATED | Building |
| INR2 | YES | INTRAMURAL FIELDS RESTROOM (NUF 2300 N U | 660 | 2006 | ACADEMIC | APPROPRIATED | Building |
| INTR | YES | INTRAMURAL FIELDS STORAGE BUILDING (HELA | 1494 | 1971 | ACADEMIC | APPROPRIATED | Building |
| IPF | YES | INDOOR PRACTICE FACILITY | 106874 | 2003 | AUXILIARY | REVENUE | Building |
| ISB | YES | IRONTON STORAGE BUILDING | 37595 | 1987 | ACADEMIC | APPROPRIATED | Building |
| ITB | YES | INFORMATION TECHNOLOGY BUILDING | 81361 | 2009 | ADMIN | APPROPRIATED | Building |
| IWRA | YES | INTRAMURAL WEST RECREATION AREA |  | 1982 | ACADEMIC | APPROPRIATED | Facility |
| JFSB | YES | SMITH (JOSEPH F) BUILDING | 317552 | 2004 | ACADEMIC | APPROPRIATED | Building |
| JFSP | YES | SMITH (JOSEPH F) BUILDING PARKING GARAGE | 111280 | 2005 | ACADEMIC | APPROPRIATED | Facility |
| JKB | YES | KNIGHT (JESSE) BUILDING | 139164 | 1960 | ACADEMIC | APPROPRIATED | Building |
| JLYP | YES | JOLLEY PROPERTY |  |  | ACADEMIC | BILLING | General Site |
| JRCB | YES | CLARK (J REUBEN) BUILDING (LAW SCHOOL) | 174970 | 1975 | ACADEMIC | APPROPRIATED | Building |
| JRCP | YES | JRCB PATIO |  | 2015 | ACADEMIC | APPROPRIATED | Facility |
| JRMC | YES | BYU JERUSALEM CENTER | 120000 | 1986 | ACADEMIC | APPROPRIATED | Building |
| JSB | YES | SMITH (JOSEPH) BUILDING | 73815 | 1991 | ACADEMIC | APPROPRIATED | Building |
| JSBP | YES | JOSEPH SMITH BUILDING PATIO |  | 1991 | ACADEMIC | APPROPRIATED | Facility |
| KMBL | YES | KIMBALL (SPENCER W) TOWER | 133849 | 1981 | ACADEMIC | APPROPRIATED | Building |
| LBAT | YES | LYTLE RANCH (BATTERY SHED) | 100 | 1995 | ACADEMIC | APPROPRIATED | Building |
| LBKH | YES | LYTLE RANCH (BUNKHOUSE) | 489 | 1995 | ACADEMIC | APPROPRIATED | Building |
| LBT2 | YES | LYTLE RANCH (BATTERY SHED 2) | 100 | 2018 | ACADEMIC | APPROPRIATED | Building |
| LDSC | YES | LDS CONFERENCE CENTER |  |  |  | CHURCH | Building |
| LDSP | YES | LDS PHILANTHROPIES BUILDING | 34467 | 2008 | ADMIN | BILLING | Building |
| LGZB | YES | LYTLE RANCH (GAZEBO) | 196 | 1995 | ACADEMIC | APPROPRIATED | Facility |
| LMH | YES | LYTLE RANCH (MANAGERS HOUSE) | 1412 | 1995 | ACADEMIC | REVENUE | Building |
| LNDC | YES | LONDON CENTER | 22544 | 1977 | ACADEMIC | REVENUE | Building |
| LPVL | YES | LYTLE RANCH (PAVILION) | 800 | 1995 | ACADEMIC | APPROPRIATED | Facility |
| LRBH | YES | LYTLE RANCH (BUNKHOUSE AND CLASSROOM) | 3223 | 2013 | ACADEMIC | APPROPRIATED | Building |
| LRMB | YES | LYTLE RANCH (RESTROOM) | 297 | 1995 | ACADEMIC | APPROPRIATED | Building |
| LRSD | YES | LYTLE RANCH (ROAD SIDE INFO DISPLAY) | 96 | 1995 | ACADEMIC | APPROPRIATED | Facility |
| LSB | YES | LIFE SCIENCES BUILDING | 269936 | 2014 | ACADEMIC | APPROPRIATED | Building |
| LSBP | YES | LIFE SCIENCES BUILDING PARKING | 89055 | 2014 | ACADEMIC | APPROPRIATED | Facility |
| LSC | YES | LIFE SCIENCE'S COMPLEX |  | 2011 | ACADEMIC | APPROPRIATED | Facility |
| LSG1 | YES | LIFE SCIENCES GREENHOUSES STORAGE SHED # | 784 | 2011 | ACADEMIC | APPROPRIATED | Building |
| LSG2 | YES | LIFE SCIENCES GREENHOUSES STORAGE SHED # | 200 | 2011 | ACADEMIC | APPROPRIATED | Building |
| LSGH | YES | LIFE SCIENCES GREENHOUSES | 21021 | 2011 | ACADEMIC | APPROPRIATED | Building |
| LSH1 | YES | LYTLE RANCH (SHOP 1) | 264 | 2018 | ACADEMIC | APPROPRIATED | Building |
| LSH2 | YES | LYTLE RANCH (SHOP 2) | 264 | 2018 | ACADEMIC | APPROPRIATED | Building |
| LSRL | YES | LIFE SCIENCES RESEARH LAB (FORMERLY HORS | 6189 | 1992 | ACADEMIC | APPROPRIATED | Building |
| LTS | YES | LYTLE RANCH TUFF SHED |  | 2010 | ACADEMIC | APPROPRIATED | Facility |
| LVES | YES | LAVELL EDWARDS STADIUM | 172688 | 1964 | AUXILIARY | REVENUE | Building |
| LWHH | YES | LYTLE RANCH (WELL HEAD HOUSE) | 20 | 1995 | ACADEMIC | APPROPRIATED | Facility |
| LYTL | YES | LYTLE NATURE PRESERVE (563) | 6088 | 1986 | ACADEMIC | APPROPRIATED | General Site |
| MARB | YES | MARTIN (THOMAS L) BUILDING | 43717 | 1969 | ACADEMIC | APPROPRIATED | Building |
| MBPF | YES | COUGAR MARCHING BAND PRACTICE FACILITY |  | 2010 | ACADEMIC | APPROPRIATED | Facility |
| MC | YES | MARRIOTT (J WILLARD) CENTER | 230305 | 1971 | AUXILIARY | APPROPRIATED | Building |
| MCA | YES | MARRIOTT CENTER ANNEX | 36931 | 2016 | AUXILIARY | REVENUE | Building |
| MCDB | YES | MCDONALD (HOWARD S) BUILDING | 18159 | 1955 | ADMIN | APPROPRIATED | Building |
| MCKB | YES | MCKAY (DAVID O) BUILDING | 80939 | 1954 | ACADEMIC | APPROPRIATED | Building |
| MHA1 | YES | GROUNDS MATERIALS HANDLING AREA |  | 1983 | ADMIN | APPROPRIATED | Facility |
| MHA2 | YES | GROUNDS MATERIALS HANDLING AREA 2 (MAPLE |  | 1990 | ADMIN | APPROPRIATED | Facility |
| MLBM | YES | BEAN (MONTE L) LIFE SCIENCE MUSEUM | 88041 | 1978 | ACADEMIC | APPROPRIATED | Building |
| MLFP | YES | MARY LOU FULTON PLAZA |  | 2005 | ACADEMIC | APPROPRIATED | Facility |
| MLRB | YES | MILLER (LARRY H.) BASEBALL STADIUM | 2210 | 2001 | AUXILIARY | REVENUE | Facility |
| MLRP | YES | MILLER PARK BUILDING | 31935 | 2001 | AUXILIARY | REVENUE | Building |
| MLRS | YES | MILLER (GAIL) SOFTBALL STADIUM | 2020 | 2001 | ACADEMIC | REVENUE | Facility |
| MOA | YES | MUSEUM OF ART | 99793 | 1993 | ACADEMIC | REVENUE | Building |
| MOAS | YES | MUSEUM OF ART SCULPTURE GARDEN |  | 2012 | ACADEMIC | REVENUE | Facility |
| MORP | YES | MORRIS CENTER EAST PATIO |  | 1964 | AUXILIARY | REVENUE | Facility |
| MP | YES | MUSEUM OF PALEONTOLOGY | 13990 | 1976 | ACADEMIC | APPROPRIATED | Building |
| MPBU | YES | MOTION PICTURE STUDIOS - BATTERY BACK-UP | 178 | 2009 | AUXILIARY | APPROPRIATED | Building |
| MPCA | YES | MOTION PICTURE STUDIOS CABIN | 374 | 1972 | ADMIN | APPROPRIATED | Building |
| MPEF | YES | MOTION PICTURE STUDIOS ELBERTA FIRE SUPP | 320 | 2011 | ADMIN | APPROPRIATED | Building |
| MPEJ | YES | MOTION PICTURE STUDIO ELBERTA JERSUALEM | 0 | 2011 | ADMIN | APPROPRIATED | Facility |
| MPES | YES | MOTION PICTURE STUDIOS ELBERTA JERUSALEM | 5700 | 2011 | ADMIN | APPROPRIATED | Building |
| MPH | YES | PROVO MISSION PRESIDENT'S HOME | 0 | 1986 | NON-INST | BILLING | Building |
| MPLG | YES | MAPLETON GROUNDS SCALE SITE (SHED) |  |  | ADMIN | APPROPRIATED | Non-Asset |
| MPS | YES | MOTION PICTURE STUDIOS | 48568 | 1958 | ADMIN | APPROPRIATED | Building |
| MPSB | YES | MOTION PICTURE STUDIOS - JUNCTION BOX | 64 | 1988 | ADMIN | APPROPRIATED | Building |
| MPSC | YES | MOTION PICTURE STUDIOS - CHEM STG | 800 | 1994 | ADMIN | APPROPRIATED | Building |
| MPSF | YES | MOTION PICTURE STUDIOS - FAR WEST | 4122 | 2004 | AUXILIARY | APPROPRIATED | Building |
| MPSG | YES | MOTION PICTURE STUDIOS GUARD HOUSE | 54 | 2001 | ADMIN | APPROPRIATED | Building |
| MPSO | YES | MOTION PICTURE STUDIOS - OPERATIONS BLDG | 2944 | 1975 | ADMIN | APPROPRIATED | Building |
| MPSP | YES | MOTION PICTURE STUDIOS 'J SMITH HOUSE' | 1480 | 1975 | ADMIN | APPROPRIATED | Building |
| MPSR | YES | MOTION PICTURE STUDIOS HOUSE - THE RANCH | 5500 | 2004 | AUXILIARY | APPROPRIATED | Building |
| MPSS | YES | MOTION PICTURE STUDIOS - SHOP | 20083 | 2001 | AUXILIARY | APPROPRIATED | Building |
| MPSU | YES | MOTION PICTURE STUDIOS - POWER DISTRIBUT | 693 | 2001 | AUXILIARY | APPROPRIATED | Building |
| MPT2 | YES | MOTION PICTURE STUDIOS - TRAILER 2 | 978 | 2006 | ADMIN | APPROPRIATED | Building |
| MRPV | YES | MORRIS CENTER PAVILION | 2007 | 1994 | AUXILIARY | REVENUE | Facility |
| MSRB | YES | MAESER (KARL G) BUILDING | 19255 | 1911 | ACADEMIC | APPROPRIATED | Building |
| MTC | YES | MISSIONARY TRAINING CENTER | 0 | 1976 | NON-INST | BILLING | General Site |
| MTCB | YES | MTC PEDESTRIAN SECURITY BOOTH | 1 | 2019 | NON-INST | APPROPRIATED | Building |
| MTCC | YES | MTC PRESIDENTS CONDO | 2383 | 2016 | NON-INST | APPROPRIATED | Building |
| MTCE | YES | MTC ENTRANCE GUARD BUILDING | 126 | 2010 | NON-INST | APPROPRIATED | Building |
| MTCG | YES | MTC FIELD & TEMPLE GROUNDS STORAGE BUILD | 1056 | 2006 | NON-INST | APPROPRIATED | Building |
| MTCM | YES | MTC SPRINKLER SYSTEM MECHANICAL BUILDING | 51 | 2009 | NON-INST | APPROPRIATED | Building |
| MTCP | YES | MTC PARKING GARAGE | 22932 | 2017 | NON-INST | APPROPRIATED | Building |
| MTCU | YES | MTC PARK |  | 1970 | NON-INST | BILLING | Meter |
| MTV | YES | MOUNT VISION TRANSMITTER BUILDING (KBYU) | 3436 | 1978 | ACADEMIC | APPROPRIATED | Building |
| NEF | YES | NORTH EAST FIELD |  | 1964 | AUXILIARY | APPROPRIATED | Facility |
| NHWF | YES | NORTH HILLSIDE WATER FEATURE |  |  | ADMIN | APPROPRIATED | Facility |
| NICB | YES | NICHOLES (JOSEPH K) BLDG (CHEM STORES) | 23920 | 1971 | ACADEMIC | APPROPRIATED | Building |
| NUF | YES | NORTH UNIVERSITY FIELD |  | 2006 | ACADEMIC | APPROPRIATED | Facility |
| NUFS | YES | NORTH UNIVERSITY FIELDS STORAGE BUILDING | 340 | 2015 | AUXILIARY | REVENUE | Building |
| OBS1 | YES | OBSERVATORY (1) - WEST MOUNTAIN | 2466 | 1981 | ACADEMIC | APPROPRIATED | Building |
| OBS2 | YES | OBSERVATORY (2) - WEST MOUNTAIN | 190 | 1996 | ACADEMIC | APPROPRIATED | Building |
| OBS3 | YES | OBSERVATORY (3) - WEST MOUNTAIN | 122 | 2008 | ACADEMIC | APPROPRIATED | Building |
| OBSH | YES | OBSERVATORY RESIDENCE - WEST MOUNTAIN | 1975 | 1982 | ACADEMIC | APPROPRIATED | Building |
| OFFC | YES | OFF-CAMPUS |  |  |  | BILLING | Non-Asset |
| ONOB | YES | OPERATING NO BLDG |  |  |  | APPROPRIATED | General Site |
| PARS | YES | PARIS HOUSING (LEASED) | 1221 |  | HOUSING | LEASED | Building |
| PAV1 | YES | PAVILION 1 | 180 | 1950 | ADMIN | APPROPRIATED | Facility |
| PBF | YES | PROVO BAY FACILITY | 1 | 2018 | ADMIN | APPROPRIATED | Facility |
| PBV1 | YES | TRAFFIC CONTROL BOOTH #1 NORTH | 81 | 1980 | ADMIN | APPROPRIATED | Building |
| PBV4 | YES | TRAFFIC CONTROL BOOTH #4 SOUTH | 53 | 1999 | ADMIN | APPROPRIATED | Building |
| PBV5 | YES | TRAFFIC CONTROL BOOTH #5 EAST | 59 | 2013 | ADMIN | APPROPRIATED | Building |
| PBV6 | YES | TRAFFIC CONTROL BOOTH #6 WEST | 63 | 2016 | ADMIN | APPROPRIATED | Building |
| PCT1 | YES | PHYSICAL PLANT COOLING TOWER 1 | 0 |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| PCT2 | YES | PHYSICAL PLANT COOLING TOWER 2 | 0 | 2015 | ADMIN | APPROPRIATED | DISTRIBUTION |
| PFNB | YES | PHYSICAL FACILITIES NORTH BUILDING | 52914 | 2015 | AUXILIARY | REVENUE | Building |
| PG12 | YES | PHYSICAL PLANT GROUNDS MATERIALS HANDLIN | 257 | 1983 | ADMIN | APPROPRIATED | Building |
| PG13 | YES | PHYSICAL PLANT GROUNDS RENTAL TOOLS STOR | 285 | 1990 | ADMIN | APPROPRIATED | Building |
| PGCE | YES | PHYSICAL PLANT GROUNDS STORAGE - MAIN CA | 294 | 2019 | ADMIN | APPROPRIATED | Building |
| PGEC | YES | PHYSICAL PLANT GROUNDS STORAGE - EAST CA | 436 | 1996 | ADMIN | APPROPRIATED | Building |
| PGHR | YES | PHYSICAL PLANT GROUNDS STORAGE - HERITAG | 389 | 2010 | ADMIN | APPROPRIATED | Building |
| PGIW | YES | PHYSICAL PLANT GROUNDS STORAGE - INTRAMU | 336 | 2019 | ADMIN | APPROPRIATED | Building |
| PGMC | YES | PHYSICAL PLANT GROUNDS OFFICE - MAIN CAM | 255 | 2009 | ADMIN | APPROPRIATED | Building |
| PGMN | YES | PHYSICAL PLANT GROUNDS STORAGE - MATERIA | 6154 | 1983 | ADMIN | APPROPRIATED | Building |
| PGMS | YES | PHYSICAL PLANT GROUNDS STORAGE - MATERIA | 5664 | 1983 | ADMIN | APPROPRIATED | Building |
| PGNC | YES | PHYSICAL PLANT GROUNDS STORAGE - NORTH C | 389 | 2010 | ADMIN | APPROPRIATED | Building |
| PGP6 | YES | PHYSICAL PLANT GROUNDS STORAGE - PARKING | 285 | 2008 | ADMIN | APPROPRIATED | Building |
| PGRC | YES | PHYSICAL PLANT GROUNDS STORAGE - ROOT CE | 507 | 1946 | ADMIN | APPROPRIATED | Building |
| PGSC | YES | PHYSICAL PLANT GROUNDS STORAGE - SOUTH C | 285 | 2008 | ADMIN | APPROPRIATED | Building |
| PGV | YES | GROUNDS MAINTENANCE SHELTER |  | 2012 | ADMIN | APPROPRIATED | Facility |
| PGWC | YES | PHYSICAL PLANT GROUNDS STORAGE - WEST CA | 814 | 1971 | ADMIN | APPROPRIATED | Building |
| PGWT | YES | PHYSICAL PLANT GROUNDS STORAGE - WYMOUNT | 613 | 1997 | HOUSING | REVENUE | Building |
| PHPL | YES | PHPL - PHYSICAL PLANT BUILDINGS |  | 1988 |  | BILLING | Non-Asset |
| PHSS | YES | PROVO HIGH SCHOOL SEMINARY | 14707 | 2018 | ACADEMIC | Investment Property | Building |
| PL01 | YES | PARKING LOT 1 (ASB PARKING) |  |  |  | APPROPRIATED | General Site |
| PL02 | YES | PARKING LOT 2 (MOA PARKING) |  |  |  | APPROPRIATED | General Site |
| PL03 | YES | PARKING LOT 3 (WSC/HFAC PARKING) |  |  |  | APPROPRIATED | General Site |
| PL04 | YES | PARKING LOT 4(S OF WSC) |  |  |  | APPROPRIATED | General Site |
| PL05 | YES | PARKING LOT 5 (WEST OF BRWB) |  |  |  | APPROPRIATED | General Site |
| PL06 | YES | PARKING LOT 6 (EAST OF MB) |  |  |  | APPROPRIATED | General Site |
| PL07 | YES | PARKING LOT 7 (BETWEEN WIDB & BNSN) |  |  |  | APPROPRIATED | General Site |
| PL09 | YES | PARKING LOT 9 (S OF JSB) |  |  |  | APPROPRIATED | General Site |
| PL11 | YES | PARKING LOT 11 (SW OF HGB) |  |  |  | APPROPRIATED | General Site |
| PL12 | YES | PARKING LOT 12 (S OF FPH) |  |  |  | APPROPRIATED | General Site |
| PL13 | YES | PARKING LOT 13 (W OF MCKB) |  |  |  | APPROPRIATED | General Site |
| PL14 | YES | PARKING LOT 14 (JFSB UNDERGROUND) |  |  |  | APPROPRIATED | General Site |
| PL15 | YES | PARKING LOT 15 (W OF FOB) |  |  |  | APPROPRIATED | General Site |
| PL16 | YES | PARKING LOT 16 (N OF JKB) |  |  |  | APPROPRIATED | General Site |
| PL18 | YES | PARKING LOT 18 (E 1230 N ST - NEAR BASKI |  |  |  | APPROPRIATED | General Site |
| PL19 | YES | PARKING LOT 19 (MARRIOTT CENTER PARKING) |  |  |  | APPROPRIATED | General Site |
| PL20 | YES | PARKING LOT 20 (MARRIOT CENTER NE PARKIN |  |  |  | APPROPRIATED | General Site |
| PL21 | YES | PARKING LOT 21 (W OF HCEB) |  |  |  | APPROPRIATED | General Site |
| PL22 | YES | PARKING LOT 22 (N OF MLBM/S OF BYUB) |  |  |  | APPROPRIATED | General Site |
| PL23 | YES | PARKING LOT 23 (E OF CSC) |  |  |  | APPROPRIATED | General Site |
| PL25 | YES | PARKING LOT 25 (HERITAGE HALLS) |  |  |  | APPROPRIATED | General Site |
| PL26 | YES | PARKING LOT 26 (E OF WSC/JRCB) |  |  |  | APPROPRIATED | General Site |
| PL27 | YES | PARKING LOT 27 (E OF SNLB) |  |  |  | APPROPRIATED | General Site |
| PL28 | YES | PARKING LOT 28 (N OF PPGO) |  |  |  | APPROPRIATED | General Site |
| PL29 | YES | PARKING LOT 29 (SE OF BRWB) |  |  |  | APPROPRIATED | General Site |
| PL30 | YES | PARKING LOT 30 (SW OF BRWB) |  |  |  | APPROPRIATED | General Site |
| PL31 | YES | PARKING LOT 31 (E OF PGP6) |  |  |  | APPROPRIATED | General Site |
| PL33 | YES | PARKING LOT 33 (E OF WTHR/PGSC) |  |  |  | APPROPRIATED | General Site |
| PL34 | YES | PARKING LOT 34 (S OF TENNIS COMPLEX) |  |  |  | APPROPRIATED | General Site |
| PL35 | YES | PARKING LOT 35 (E OF AKH) |  |  |  | APPROPRIATED | General Site |
| PL36 | YES | PARKING LOT 36 (S OF IPF) |  |  |  | APPROPRIATED | General Site |
| PL37 | YES | PARKING LOT 37 (N OF IPF/IPF PARKING) |  |  |  | APPROPRIATED | General Site |
| PL38 | YES | PARKING LOT 38 (E OF IPF) |  |  |  | APPROPRIATED | General Site |
| PL39 | YES | PARKING LOT 39 (W OF RB/RB PARKING) |  |  |  | APPROPRIATED | General Site |
| PL40 | YES | PARKING LOT 40 (TANNER BUILDING PARKING) |  |  |  | APPROPRIATED | General Site |
| PL41 | YES | PARKING LOT 41 (HELAMAN HALLS PARKING) |  |  |  | APPROPRIATED | General Site |
| PL42 | YES | PARKING LOT 42 (ELLSWORTH PARKING LOT) |  |  |  | APPROPRIATED | General Site |
| PL43 | YES | PARKING LOT 43 (WYVIEW PARK) |  |  |  | APPROPRIATED | General Site |
| PL44 | YES | PARKING LOT 44 (WEST INTRAMURAL FIELDS P |  |  |  | APPROPRIATED | General Site |
| PL45 | YES | PARKING LOT 45 (WEST STADIUM PARKING) |  |  |  | APPROPRIATED | General Site |
| PL46 | YES | PARKLING LOT 46 (AROUND BYUW CAMPUS) |  |  |  | Investment Property | General Site |
| PL47 | YES | PARKING LOT 47 (NORTHEAST OF STADIUM) |  |  |  | APPROPRIATED | General Site |
| PL48 | YES | PARKING LOT 48 (SOUTH STADIUM PARKING) |  |  |  | APPROPRIATED | General Site |
| PL49 | YES | PARKING LOT 49 (EAST STADIUM PARKING) |  |  |  | APPROPRIATED | General Site |
| PL50 | YES | PARKING LOT 50 (E OF UPB/W OF AXMB) |  |  |  | APPROPRIATED | General Site |
| PL51 | YES | PARKING LOT 51 (MTC) |  |  |  | APPROPRIATED | General Site |
| PL52 | YES | PARKING LOT 52 (WYMOUNT TERRACE) |  |  |  | APPROPRIATED | General Site |
| PL53 | YES | PARKING LOT 53 (TAYLOR BUILDING PARKING) |  |  |  | APPROPRIATED | General Site |
| PL54 | YES | PARKING LOT 54 (B67) |  |  |  | APPROPRIATED | General Site |
| PL56 | YES | PARKING LOT 56 (SOUTH HILL PARKING) |  |  |  | APPROPRIATED | General Site |
| PL57 | YES | PARKING LOT 57 (ALLN PARKING) |  |  |  | APPROPRIATED | General Site |
| PL58 | YES | PARKING LOT 58 (ITB PARKING) |  |  |  | APPROPRIATED | General Site |
| PL59 | YES | PARKING LOT 59 (S OF LSB) |  |  |  | APPROPRIATED | General Site |
| PL60 | YES | PARKING LOT 60 (STUDENT HEALTH CENTER PA |  |  |  | APPROPRIATED | General Site |
| PL61 | YES | PARKING LOT 61 (NORTH INTRAMURAL FIELDS |  |  |  | APPROPRIATED | General Site |
| PL62 | YES | PARKING LOT 62 (NORTH OF STADIUM) |  |  |  | Investment Property | General Site |
| PPBH | YES | PHYSICAL PLANT BAG HOUSE | 1518 | 1993 | ADMIN | APPROPRIATED | Building |
| PPC1 | YES | MARB CHILLER | 4485 | 1946 | ADMIN | APPROPRIATED | Building |
| PPC2 | YES | CONF CHILLER | 0 | 2016 | ADMIN | APPROPRIATED | Meter |
| PPCC | YES | PHYSICAL PLANT COOLING TOWER CHEMICALS | 50 | 2015 | ADMIN | APPROPRIATED | Building |
| PPCH | YES | PHYSICAL PLANT CENTRAL HEATING PLANT | 52010 | 1946 | ADMIN | APPROPRIATED | Building |
| PPCW | YES | CENTRAL CHILLED WATER PLANT | 12644 | 1971 | ADMIN | APPROPRIATED | Building |
| PPE1 | YES | PHPL~ ELECTRICAL SUBSTATION #1 | 1238 | 1962 | ADMIN | APPROPRIATED | Building |
| PPE2 | YES | ELECTRICAL EQUIPMENT STORAGE | 730 | 1962 | ADMIN | APPROPRIATED | Building |
| PPE3 | YES | PHPL~ ELECTRICAL SUBSTATION #3 | 718 | 1962 | ADMIN | APPROPRIATED | Building |
| PPE4 | YES | PHPL~ ELECTRICAL SUBSTATION #4 | 1008 | 1988 | ADMIN | APPROPRIATED | DISTRIBUTION |
| PPE5 | YES | PHPL~ HTHW EQUIPMENT VAULT | 140 | 1964 | ADMIN | APPROPRIATED | DISTRIBUTION |
| PPE6 | YES | PHPL GENERATOR ROOM #6 | 204 | 1996 | ADMIN | APPROPRIATED | Building |
| PPET | YES | PHPL~ ELECTRICAL VAULT - SFH TENNIS CRTS | 192 | 1994 | ADMIN | APPROPRIATED | DISTRIBUTION |
| PPEV | YES | PHPL~ ELEC. SWITCHING VAULT | 623 | 1960 | ADMIN | APPROPRIATED | DISTRIBUTION |
| PPG1 | YES | ASB GENERATOR VAULT | 431 | 1961 | ADMIN | APPROPRIATED | Building |
| PPG2 | YES | GENERATOR ROOMS IN BNSN C048B/D | 1202 | 1995 | ADMIN | APPROPRIATED | Building |
| PPG3 | YES | GENERATOR FACILITY SOUTH OF HR27 | 1887 | 2012 | ADMIN | APPROPRIATED | Facility |
| PPG4 | YES | GENERATOR FACILITY SOUTH OF HR07 | 1266 | 2013 | ADMIN | APPROPRIATED | Facility |
| PPGC | YES | PHYSICAL PLANT GROUNDS CHANGE BUILDING | 2044 | 1983 | ADMIN | APPROPRIATED | Building |
| PPGE | YES | PHYSICAL PLANT GROUNDS EQUIPMENT BUILDIN | 6885 | 1973 | ADMIN | APPROPRIATED | Building |
| PPGH | YES | PHYSICAL PLANT GREENHOUSE | 8363 | 1963 | ADMIN | APPROPRIATED | Building |
| PPGL | YES | PHPL~ GROUNDS LOADER SHELTER | 7342 | 2003 | ADMIN | APPROPRIATED | Facility |
| PPGM | YES | PHPL~ GROUNDS MAINTENANCE SHELTER | 3680 | 2003 | ADMIN | APPROPRIATED | Facility |
| PPGO | NO | PHYSICAL PLANT GROUNDS OFFICE | 2596 | 1963 | ADMIN | APPROPRIATED | Discontinued |
| PPGT | YES | PHYSICAL PLANT GROUNDS TRUCK GARAGE | 5219 | 1982 | ADMIN | APPROPRIATED | Building |
| PPMO | YES | PHYSICAL PLANT MOTOR POOL OFFICE BUILDIN | 662 | 1971 | ADMIN | APPROPRIATED | Building |
| PPMV | YES | PHYSICAL PLANT MOTOR POOL VEHICLE SHELTE | 5358 | 1971 | ADMIN | APPROPRIATED | Facility |
| PPMW | YES | PHYSICAL PLANT MOTOR POOL CAR WASH | 943 | 1971 | ADMIN | APPROPRIATED | Building |
| PPPH | YES | PHYSICAL PLANT PUMP HOUSE (CANAL) | 101 | 1992 | ADMIN | APPROPRIATED | Building |
| PPS1 | YES | PHYSICAL PLANT GENERAL STORAGE BUILDING | 12866 | 1982 | ADMIN | APPROPRIATED | Building |
| PPS2 | YES | PHYSICAL PLANT GENERAL STORAGE BUILDING | 12863 | 1982 | ADMIN | APPROPRIATED | Building |
| PPS3 | YES | PHYSICAL PLANT GENERAL STORAGE BUILDING | 8000 | 1982 | ADMIN | APPROPRIATED | Building |
| PPS5 | YES | PHYSICAL PLANT STORAGE BUILDING (MOVING) | 1480 | 1987 | ACADEMIC | APPROPRIATED | Building |
| PPSS | YES | PHYSICAL PLANT SERVICE STATION | 1958 | 1980 | ADMIN | APPROPRIATED | Building |
| PPT3 | YES | PHPL~ PHONE SYSTEM BUILDING #3 | 455 | 1988 | ADMIN | APPROPRIATED | Building |
| PPT4 | YES | PHPL~ PHONE SYSTEM BUILDING #4 | 360 | 1988 | AUXILIARY | APPROPRIATED | Building |
| PPV6 | YES | PHPL~ HTHW EQUIPMENT VAULT #6 | 535 | 1964 | ADMIN | APPROPRIATED | DISTRIBUTION |
| PPV8 | YES | PHPL~ HTHW EQUIPMENT VAULT #8 | 216 | 1968 | ADMIN | APPROPRIATED | DISTRIBUTION |
| PPWL | YES | PHPL~ WELL PUMP BUILDING | 308 | 1979 | ADMIN | APPROPRIATED | Building |
| PPX1 | YES | HEAT EXCHANGER ROOM OUTSIDE TNRB | 364 |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| PPX2 | YES | MCKB HEAT EXCHANGE ROOM |  |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| PPX3 | YES | RB/SFH TUNNEL/HEAT EXCAHNGE |  |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| PPX4 | YES | WSC/JRCB HEAT EXCHANGE ROOM | 2187 | 2013 | ADMIN | APPROPRIATED | DISTRIBUTION |
| PPX5 | YES | LSB HEAT EXCHANGE ROOM | 1575 | 2013 | ADMIN | APPROPRIATED | DISTRIBUTION |
| PTVD | YES | PHYSICS TV DISH BLDG (SPANISH FORK FARM) | 225 | 1981 | ACADEMIC | APPROPRIATED | Building |
| QBEL | YES | CARILLON TOWER QUAD |  | 1975 | ACADEMIC | APPROPRIATED | Facility |
| QBOT | YES | HARRISON (BERTRAND F) ARBORETUM AND BOTA |  | 1945 | ACADEMIC | APPROPRIATED | Facility |
| QBS | YES | BRIGHAM SQUARE |  | 1999 | ACADEMIC | APPROPRIATED | Facility |
| QCGR | YES | COUGAR QUAD |  | 2015 | ACADEMIC | APPROPRIATED | Facility |
| QFDW | YES | FIELDING WAY |  |  | ACADEMIC | APPROPRIATED | Facility |
| QHL | YES | HELAMAN HALLS QUAD |  | 1958 | HOUSING | APPROPRIATED | Facility |
| QJFS | YES | JOSEPH F. SMITH BUILDING QUAD |  | 2005 | ACADEMIC | APPROPRIATED | Facility |
| QKT | YES | KIMBALL TOWER QUAD |  | 1981 | ACADEMIC | APPROPRIATED | Facility |
| QLL | YES | LEE LANE QUAD |  | 1999 | ACADEMIC | APPROPRIATED | Facility |
| QMG | YES | MARIGOLD QUAD |  | 1970 | ACADEMIC | APPROPRIATED | Facility |
| QMSR | YES | MAESER BUILDING QUAD |  | 1999 | ACADEMIC | APPROPRIATED | Facility |
| QRB | YES | RICHARDS BUILDING QUAD |  | 1965 | ACADEMIC | APPROPRIATED | Facility |
| QSC | YES | SOUTH CAMPUS QUAD |  | 2018 | ACADEMIC | APPROPRIATED | Facility |
| QTC | YES | TENNIS COURT FACILITY QUAD (BY SFH) |  | 1990 | ACADEMIC | APPROPRIATED | Facility |
| QWW | YES | WILKINSON WAY QUAD |  | 1999 | ACADEMIC | APPROPRIATED | Facility |
| R1 | YES | (R1) NYE (HARRIET M) BLDG (RESIDENCE) | 42114 | 1976 | NON-INST | APPROPRIATED | Building |
| R2 | YES | (R2) STEVENSON (EDWARD) BLDG (RESIDENCE) | 42114 | 1977 | NON-INST | APPROPRIATED | Building |
| R3 | YES | (R3) SNOW (ERASTUS) BLDG (RESIDENCE) | 42114 | 1976 | NON-INST | APPROPRIATED | Building |
| R4 | YES | (R4) STANDING (JOSEPH) BLDG (RESIDENCE) | 42114 | 1977 | NON-INST | APPROPRIATED | Building |
| R5 | YES | (R5) PRATT (REY L) BLDG (RESIDENCE) | 42114 | 1976 | NON-INST | APPROPRIATED | Building |
| R6 | YES | (R6) PRATT (ADDISON) BLDG (RESIDENCE) | 42114 | 1978 | NON-INST | APPROPRIATED | Building |
| R7 | YES | (R7) BOOTH (WILFORD) BLDG (RESIDENCE) | 42114 | 1977 | NON-INST | APPROPRIATED | Building |
| R8 | YES | (R8) JONES (DAN) BLDG (RESIDENCE) | 42114 | 1978 | NON-INST | APPROPRIATED | Building |
| R9 | YES | (R9) RICHARDS (LEGRAND) BLDG (RESIDENCE) | 80377 | 1993 | NON-INST | APPROPRIATED | Building |
| RB | YES | RICHARDS (STEPHEN L) BUILDING | 256361 | 1965 | ACADEMIC | APPROPRIATED | Building |
| RBF | YES | RICHARDS BUILDING FIELDS |  | 1970 | ACADEMIC | APPROPRIATED | Facility |
| RGDR | YES | RIVERSIDE GOLF DRIVING RANGE |  | 1989 | ACADEMIC | APPROPRIATED | Facility |
| RMB | YES | BYU RISK MANAGEMENT | 6638 | 1964 | ACADEMIC | APPROPRIATED | Building |
| ROTC | YES | WELLS (DANIEL H) BUILDING (ROTC) | 15188 | 1968 | ACADEMIC | APPROPRIATED | Building |
| RPPV | YES | RIVER PARK PAVILION | 1594 | 1983 | ADMIN | APPROPRIATED | Facility |
| RPRA | YES | RIVER PARK RECREATION AREA |  | 1981 | ADMIN | APPROPRIATED | Facility |
| RPRR | YES | RIVER PARK RESTROOM | 189 | 1981 | AUXILIARY | APPROPRIATED | Building |
| RRM | YES | SFH TENNIS STORAGE AND RESTROOM BUILDING | 601 | 1959 | ACADEMIC | APPROPRIATED | Building |
| S1 | YES | (S1) WOODRUFF (WILFORD) BLDG (ADMINISTRA | 119010 | 1976 | NON-INST | APPROPRIATED | Building |
| S2 | YES | (S2) HAMBLIN (JACOB) BLDG (COUPLE RESIDE | 67544 | 1976 | NON-INST | APPROPRIATED | Building |
| S3 | YES | (S3) SNOW (LORENZO) BLDG (MULTI-PURPOSE) | 35794 | 1993 | NON-INST | APPROPRIATED | Building |
| SAB | YES | STUDENT ATHLETE BUILDING | 122540 | 2003 | AUXILIARY | REVENUE | Building |
| SC50 | YES | GREENHOUSE POTS STORAGE CONTAINER (BLUE) | 88 |  | ADMIN | APPROPRIATED | Non-Asset |
| SC51 | YES | GROUNDS FERTILIZER STORAGE CONTAINER #1 | 319 |  | ADMIN | APPROPRIATED | Non-Asset |
| SC52 | YES | GROUNDS CALCIUM CHLORIDE STORAGE CONTAIN | 319 |  | ADMIN | APPROPRIATED | Non-Asset |
| SC53 | YES | GROUNDS RECYCLE STORAGE CONTAINER #1 (GR | 319 |  | ADMIN | APPROPRIATED | Non-Asset |
| SC54 | YES | GROUNDS RECYCLE STORAGE CONTAINER #2 (GR | 319 |  | ADMIN | APPROPRIATED | Non-Asset |
| SC55 | YES | GROUNDS NEW STORAGE CONTAINER (RED) | 158 |  | ADMIN | APPROPRIATED | Non-Asset |
| SC56 | YES | LANDSCAPE STORAGE CONTAINER NEXT TO ST08 | 94 |  | ADMIN | APPROPRIATED | Non-Asset |
| SC57 | YES | GROUNDS CONCRETE BLANKET/TARP STORAGE CO | 94 |  | ADMIN | APPROPRIATED | Non-Asset |
| SC58 | YES | GROUNDS SPRINKLER STORAGE CONTAINER #1 ( | 89 |  | ADMIN | APPROPRIATED | Non-Asset |
| SC59 | YES | GROUNDS SPRINKLER CONTAINER #2 (BLUE) | 89 |  | ADMIN | APPROPRIATED | Non-Asset |
| SC60 | YES | PHYSICAL FACILITIES STORAGE CONTAINER | 320 |  | ACADEMIC | APPROPRIATED | Non-Asset |
| SC61 | YES | SF FARM ARMY CHEMICAL STORAGE CONTAINER | 1 |  | ACADEMIC | APPROPRIATED | Non-Asset |
| SC62 | YES | ELECTRIC SHOP STORAGE CONTAINER (TAN) | 320 |  | ADMIN | APPROPRIATED | Non-Asset |
| SC63 | YES | GROUNDS STORAGE CONTAINER FOR PAVER EQUI | 160 |  | ADMIN | APPROPRIATED | Non-Asset |
| SC64 | YES | GEOLOGY FLAMMABLE STORAGE | 160 | 2016 | ADMIN | APPROPRIATED | Non-Asset |
| SC65 | YES | MOVING STORAGE CONTAINER | 1 |  | ADMIN | APPROPRIATED | Non-Asset |
| SCST | YES | SOUTH CAMPUS STREAM AND TRAIL |  | 2010 | ADMIN | APPROPRIATED | Facility |
| SERA | YES | SOUTHEAST RECREATION AREA |  | 2008 | AUXILIARY | APPROPRIATED | Facility |
| SF24 | YES | SPANISH FORK FARM (STANDING WELL) | 120 | 1981 | NON-INST | Investment Property | Building |
| SF54 | YES | SPANISH FORK FARM (SMALL WELL) | 245 | 1981 | NON-INST | Investment Property | Building |
| SF55 | YES | SPANISH FORK FARM (LARGE WELL) | 122 | 1981 | NON-INST | Investment Property | Building |
| SFF | YES | SPANISH FORK FARM |  |  | NON-INST | REVENUE | General Site |
| SFH | YES | SMITH (GEORGE ALBERT) FIELDHOUSE | 243250 | 1951 | ACADEMIC | APPROPRIATED | Building |
| SFHL | YES | SMITH FIELD HOUSE LAUNDRY |  |  |  | BILLING | Meter |
| SFLD | YES | SFH SOUTH FIELD | 2140 | 1999 | AUXILIARY | REVENUE | Facility |
| SFTB | YES | SOUTH FIELD TICKET BOOTH | 200 | 2008 | AUXILIARY | REVENUE | Building |
| SH01 | YES | SPANISH FORK FARM (HOUSE) | 3109 | 1981 | NON-INST | Investment Property | Building |
| SH20 | YES | SPANISH FORK FARM (CARPORT - SH01) | 100 | 1985 | NON-INST | Investment Property | Building |
| SHC | YES | STUDENT HEALTH CENTER | 38899 | 1998 | AUXILIARY | REVENUE | Building |
| SHCP | YES | SPRING HAVEN CAR PORT | 2000 | 1999 | AUXILIARY | APPROPRIATED | Building |
| SHGS | YES | SPRING HAVEN GENERAL SITE |  | 1999 | AUXILIARY | APPROPRIATED | General Site |
| SHGZ | YES | SPRING HAVEN GAZEBO |  | 1999 | AUXILIARY | APPROPRIATED | Facility |
| SHPH | YES | SPRING HAVEN PUMP HOUSE | 156 | 1999 | AUXILIARY | APPROPRIATED | Building |
| SL80 | YES | SPANISH FORK FARM (PUMPHOUSE) | 52 | 1981 | NON-INST | Investment Property | Building |
| SL83 | YES | SPANISH FORK FARM (STORAGE) | 18200 | 1981 | ACADEMIC | APPROPRIATED | Building |
| SL84 | YES | SPANISH FORK FARM (MONITORING STATION-TU | 128 | 2009 | ACADEMIC | APPROPRIATED | Building |
| SLC | YES | SALT LAKE CENTER (BYU CONTINUING EDUCATI | 68300 | 2007 | ACADEMIC | CHURCH | Building |
| SNLB | YES | SNELL (WILLIAM H) BUILDING | 37796 | 1959 | ACADEMIC | APPROPRIATED | Building |
| SPHV | YES | SPRING HAVEN CABIN | 18036 | 1999 | AUXILIARY | APPROPRIATED | Building |
| ST01 | YES | GROUNDS RENTAL TOOLS STORAGE SHED (GREEN | 82 | 2009 | ADMIN | APPROPRIATED | Non-Asset |
| ST02 | YES | ATHLETICS SHED/WOMEN'S SOCCER STORAGE (B | 80 | 1986 | ADMIN | APPROPRIATED | Non-Asset |
| ST03 | YES | GROUNDS FLOWER SHED (TAN) | 83 | 2019 | ADMIN | APPROPRIATED | Non-Asset |
| ST04 | YES | SCHOOL OF TECHNOLOGY SHED EAST OF SNLB ( | 194 |  | ACADEMIC | APPROPRIATED | Non-Asset |
| ST05 | YES | ATHLETICS SHED WEST OF TRACK (WHITE) | 102 |  | ADMIN | APPROPRIATED | Non-Asset |
| ST06 | YES | RECREATION EQUIPMENT STORAGE SHED (WHITE | 65 | 1998 | ACADEMIC | APPROPRIATED | Non-Asset |
| ST07 | YES | GROUNDS OFFICE TABLE STORAGE SHED (BROWN | 90 | 2013 | ADMIN | APPROPRIATED | Non-Asset |
| ST08 | YES | MHA LANDSCAPE SHED | 104 |  | ADMIN | APPROPRIATED | Non-Asset |
| ST09 | YES | GROUNDS CONCRETE SUPPLIES STORAGE | 104 |  | ADMIN | APPROPRIATED | Non-Asset |
| ST10 | YES | GROUNDS SHED (BLUE) | 67 |  | ADMIN | APPROPRIATED | Non-Asset |
| ST11 | YES | GROUNDS RECYCLING SHED (WHITE) | 346 |  | ADMIN | APPROPRIATED | Non-Asset |
| ST12 | YES | GROUNDS STORAGE (TAN) | 110 |  | ADMIN | APPROPRIATED | Non-Asset |
| ST13 | YES | GROUNDS PLASTIC TOOL STORAGE SHED (LIGHT | 29 |  | ADMIN | APPROPRIATED | Non-Asset |
| ST14 | YES | GRNHSE STORAGE SHED (BROWN) | 83 |  | ADMIN | APPROPRIATED | Non-Asset |
| ST15 | YES | SPANISH FORK FARM ADDITIONAL STORAGE #1 | 115 |  | ACADEMIC | APPROPRIATED | Non-Asset |
| ST16 | YES | SPANISH FORK FARM ADDITIONAL STORAGE #2 | 115 |  | ACADEMIC | APPROPRIATED | Non-Asset |
| ST17 | YES | SPANISH FORK FARM STORAGE SHED (YELLOW) | 1 |  | ACADEMIC | APPROPRIATED | Non-Asset |
| ST18 | YES | GROUNDS STORAGE SHED FOR FOOTBALL | 100 |  |  | APPROPRIATED | Non-Asset |
| ST19 | YES | ATHLETICS SHED/MEN'S SOCCER STORAGE (BLU | 80 | 1983 | ADMIN | APPROPRIATED | Non-Asset |
| ST20 | YES | ASPEN GROVE EQUIPMENT STORAGE SHED (BROW | 120 |  | NON-INST | REVENUE | Non-Asset |
| ST21 | YES | ASPEN GROVE STORAGE SHED (AFLA & AFLB) ( | 1 |  | NON-INST | REVENUE | Non-Asset |
| ST23 | YES | ASPEN GROVE LINEN STORAGE FOR ELK POINT | 1 |  | NON-INST | REVENUE | Non-Asset |
| ST24 | YES | ASPEN GROVE LINEN STORAGE FOR ELK POINT | 1 |  | NON-INST | REVENUE | Non-Asset |
| ST25 | YES | ASPEN GROVE PLAYHOUSE TRAIN SHED (RED) | 1 |  | NON-INST | REVENUE | Non-Asset |
| ST26 | YES | ASPEN GROVE ROPES COURSE EQUIPMENT STORA | 1 |  | NON-INST | REVENUE | Non-Asset |
| ST27 | YES | ASPEN GROVE COWBOY VILLAGE SHED | 1 |  | NON-INST | REVENUE | Non-Asset |
| ST28 | YES | ASPEN GROVE LASER TAG EQUIPMENT STORAGE | 1 |  | NON-INST | REVENUE | Non-Asset |
| ST29 | YES | ASPEN GROVE TALL PINES STORAGE SHED #1 | 1 |  | NON-INST | REVENUE | Non-Asset |
| ST30 | YES | ASPEN GROVE ARCHERY RANGE EQUIPMENT (BRO | 1 |  | NON-INST | REVENUE | Non-Asset |
| ST31 | YES | ASPEN GROVE ROPES COURSE SHED | 276 |  | NON-INST | REVENUE | Non-Asset |
| ST32 | YES | Y MOUNTAIN STORAGE BUNKER | 32 |  | ADMIN | APPROPRIATED | Non-Asset |
| ST33 | YES | LYTLE RANCH (FEED STORAGE SHED) | 64 | 2018 | ACADEMIC | APPROPRIATED | Non-Asset |
| ST34 | YES | LYTLE RANCH (FUEL STORAGE SHED) | 64 | 2018 | ACADEMIC | APPROPRIATED | Non-Asset |
| ST70 | NO | BYU WEST CAMPUS STORAGE SHED | 198 | 2018 | ACADEMIC | Investment Property | Discontinued |
| ST71 | YES | BYU WEST CAMPUS STORAGE SHED | 175 | 2018 | ADMIN | Investment Property | Non-Asset |
| ST72 | YES | BYU WEST CAMPUS STORAGE SHED | 81 | 2018 | ADMIN | Investment Property | Non-Asset |
| ST73 | YES | BYU WEST CAMPUS STORAGE SHED | 87 | 2018 | ADMIN | Investment Property | Non-Asset |
| ST74 | YES | BYU WEST CAMPUS STORAGE SHED | 112 | 2018 | ADMIN | Investment Property | Non-Asset |
| ST75 | NO | BYU WEST CAMPUS STORAGE SHED | 88 | 2018 | ADMIN | Investment Property | Discontinued |
| ST76 | NO | BYU WEST CAMPUS STORAGE SHED | 121 | 2018 | ADMIN | Investment Property | Discontinued |
| ST77 | YES | BYU WEST CAMPUS STORAGE SHED | 341 | 2018 | ADMIN | Investment Property | Non-Asset |
| ST78 | YES | BYU WEST CAMPUS STORAGE SHED | 52 | 2018 | ADMIN | Investment Property | Non-Asset |
| STTG | YES | STADIUM TICKET BOOTH GUESTS | 91 | 1983 | AUXILIARY | REVENUE | Building |
| STTW | YES | STADIUM TICKET BOOTH WEST | 316 | 1982 | AUXILIARY | REVENUE | Building |
| T1 | YES | (T1) COWLEY (MATTHEW) BLDG (TEACHING) | 42114 | 1976 | NON-INST | APPROPRIATED | Building |
| T2 | YES | (T2) PRATT (PARLEY P) BLDG (MIXED USE) | 89725 | 1993 | NON-INST | APPROPRIATED | Building |
| T3 | YES | T3 | 133315 | 2017 | NON-INST | APPROPRIATED | Building |
| T4 | YES | T4 | 234697 | 2017 | NON-INST | APPROPRIATED | Building |
| TBLK | YES | TUNNEL BLACK |  |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| TBLU | YES | TUNNEL BLUE | 0 |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| TBRN | YES | TUNNEL BROWN |  |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| TC01 | YES | TENNIS SCHEDULING BLDG - SFH OUTDOOR TEN | 96 | 1994 | ACADEMIC | APPROPRIATED | Building |
| TC02 | YES | TENNIS STORAGE - SFH OUTDOOR TENNIS COUR | 192 | 1994 | ACADEMIC | APPROPRIATED | Building |
| TC03 | YES | TENNIS STORAGE - SFH OUTDOOR TENNIS COUR | 192 | 1994 | ACADEMIC | APPROPRIATED | Building |
| TC04 | YES | TENNIS STORAGE - SFH OUTDOOR TENNIS COUR | 192 | 1994 | ACADEMIC | APPROPRIATED | Building |
| TCB | YES | TENNIS COURTS BUILDING | 29367 | 1968 | ACADEMIC | APPROPRIATED | Building |
| TCF | YES | OUTDOOR TENNIS COURT FACILITY |  | 1959 | ACADEMIC | APPROPRIATED | Facility |
| TGRN | YES | TUNNEL GREEN |  |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| TIMP | YES | TIMP LODGE | 8328 | 1971 | AUXILIARY | REVENUE | Building |
| TLRA | YES | TAYLOR BUILDING ANNEX (FORMER SEMINARY B | 3191 | 2016 | ADMIN | APPROPRIATED | Building |
| TLRB | YES | TAYLOR (JOHN) BUILDING (COMPREHENSIVE CL | 34549 | 1980 | ACADEMIC | APPROPRIATED | Building |
| TMCB | YES | TALMAGE (JAMES E) MATH SCIENCES/COMPUTER | 158696 | 1971 | ACADEMIC | APPROPRIATED | Building |
| TMPL | YES | PROVO TEMPLE | 115000 |  |  | CHURCH | Building |
| TNRB | YES | TANNER (N ELDON) BUILDING | 224901 | 1983 | ACADEMIC | APPROPRIATED | Building |
| TNRP | YES | TANNER (N ELDON) BUILDING PARKING GARAGE | 94386 | 2008 | ACADEMIC | APPROPRIATED | Facility |
| TNRW | YES | TANNER BUILDING WEST ADDITION |  | 2008 |  | BILLING | Meter |
| TORG | YES | TUNNEL ORANGE |  |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| TPLE | YES | TUNNEL PURPLE |  |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| TPNK | YES | TUNNEL PINK |  |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| TPVN | YES | TENNIS PAVILION | 924 | 1972 | ACADEMIC | APPROPRIATED | Facility |
| TRAK | YES | CLARENCE F ROBISON TRACK AND FIELD COMPL |  | 1982 | ACADEMIC | APPROPRIATED | Facility |
| TRED | YES | TUNNEL RED | 0 |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| TRPB | YES | TRACK PRESS BOX | 1883 | 1982 | ACADEMIC | APPROPRIATED | Building |
| TRPN | YES | TRACK PAVILION NORTH | 4636 | 2003 | ACADEMIC | APPROPRIATED | Facility |
| TRPS | YES | TRACK PAVILION SOUTH | 597 | 1982 | ACADEMIC | APPROPRIATED | Facility |
| TRRR | YES | TRACK RESTROOM BUILDING | 750 | 1987 | ACADEMIC | APPROPRIATED | Building |
| TRTB | YES | TRACK TICKET BOOTH | 128 | 2017 | ACADEMIC | APPROPRIATED | Building |
| TUNL | YES | CAMPUS TUNNEL SYSTEM | 198707 |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| TWHT | YES | TUNNEL WHITE |  |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| TYEL | YES | TUNNEL YELLOW | 0 |  | ADMIN | APPROPRIATED | DISTRIBUTION |
| ULB | YES | UNIVERSITY LAUNDRY BUILDING | 42502 | 2015 | AUXILIARY | REVENUE | Building |
| UPB | YES | UNIVERSITY PRESS BUILDING | 70740 | 1968 | ADMIN | APPROPRIATED | Building |
| UPBS | YES | UNIVERSITY POLICE BIKE STORAGE | 252 | 2019 | ADMIN | APPROPRIATED | Building |
| UPC | YES | UNIVERSITY PARKWAY CENTER | 48601 | 1999 | ACADEMIC | APPROPRIATED | Building |
| UPH1 | YES | UTILITIES PUMP HOUSE SOUTH HILL | 200 | 2010 | ADMIN | APPROPRIATED | DISTRIBUTION |
| USB | YES | UNIVERSITY SERVICES BUILDING | 80153 | 1968 | AUXILIARY | REVENUE | Building |
| VNNA | YES | VIENNA HOUSING (LEASED) | 1276 |  | HOUSING | LEASED | Building |
| W02A | YES | WYMOUNT TERRACE (W02A) BENNION (SAMUEL O | 24816 | 1962 | HOUSING | REVENUE | Building |
| W02B | YES | WYMOUNT TERRACE (W02B) IVINS (ANTOINE R) | 12764 | 1962 | HOUSING | REVENUE | Building |
| W02C | YES | WYMOUNT TERRACE (W02C) KIMBALL (J GOLDEN | 24816 | 1962 | HOUSING | REVENUE | Building |
| W02D | YES | WYMOUNT TERRACE (W02D) ROBERTS (BRIGHAM | 9768 | 1962 | HOUSING | REVENUE | Building |
| W03A | YES | WYMOUNT TERRACE (W03A) WILLIAMS (HELEN S | 25872 | 1963 | HOUSING | REVENUE | Building |
| W04A | YES | WYMOUNT TERRACE (W04A) NIBLEY (CHARLES W | 24816 | 1963 | HOUSING | REVENUE | Building |
| W04B | YES | WYMOUNT TERRACE (W04B) REYNOLDS (GEORGE) | 9768 | 1963 | HOUSING | REVENUE | Building |
| W04C | YES | WYMOUNT TERRACE (W04C) WIRTHLIN (JOSEPH | 24816 | 1963 | HOUSING | REVENUE | Building |
| W04D | YES | WYMOUNT TERRACE (W04D) YOUNG (LEVI EDGAR | 12764 | 1963 | HOUSING | REVENUE | Building |
| W05A | YES | WYMOUNT TERRACE (W05A) CLAYTON (WILLIAM) | 24816 | 1963 | HOUSING | REVENUE | Building |
| W05B | YES | WYMOUNT TERRACE (W05B) MCCLELLAN (JOHN O | 9768 | 1963 | HOUSING | REVENUE | Building |
| W05C | YES | WYMOUNT TERRACE (W05C) PRATT (ORSON) | 24816 | 1963 | HOUSING | REVENUE | Building |
| W05D | YES | WYMOUNT TERRACE (W05D) STEPHENS (EVAN) | 12764 | 1963 | HOUSING | REVENUE | Building |
| W06A | YES | WYMOUNT TERRACE (W06A) DIXON (HENRY ALDO | 19932 | 1963 | HOUSING | REVENUE | Building |
| W06B | YES | WYMOUNT TERRACE (W06B) HAMBLIN (JACOB) | 9768 | 1963 | HOUSING | REVENUE | Building |
| W06C | YES | WYMOUNT TERRACE (W06C) SUTHERLAND (GEORG | 19932 | 1963 | HOUSING | REVENUE | Building |
| W06D | YES | WYMOUNT TERRACE (W06D) SWENSEN (JOHN C) | 12764 | 1963 | HOUSING | REVENUE | Building |
| W08A | YES | WYMOUNT TERRACE (W08A) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W08B | YES | WYMOUNT TERRACE (W08B) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W08C | YES | WYMOUNT TERRACE (W08C) | 9436 | 1979 | HOUSING | REVENUE | Building |
| W08D | YES | WYMOUNT TERRACE (W08D) | 9436 | 1979 | HOUSING | REVENUE | Building |
| W08E | YES | WYMOUNT TERRACE (W08E) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W08F | YES | WYMOUNT TERRACE (W08F) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W09A | YES | WYMOUNT TERRACE (W09A) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W09B | YES | WYMOUNT TERRACE (W09B) | 9436 | 1979 | HOUSING | REVENUE | Building |
| W09C | YES | WYMOUNT TERRACE (W09C) | 9436 | 1979 | HOUSING | REVENUE | Building |
| W09D | YES | WYMOUNT TERRACE (W09D) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W0L1 | YES | WYMOUNT TERRACE LAUNDRY BUILDING #1 | 1531 | 1962 | HOUSING | REVENUE | Building |
| W0L2 | YES | WYMOUNT TERRACE LAUNDRY BUILDING #2 | 1531 | 1962 | HOUSING | REVENUE | Building |
| W0L3 | YES | WYMOUNT TERRACE LAUNDRY BUILDING #3 | 1156 | 2015 | HOUSING | REVENUE | Building |
| W0L4 | YES | WYMOUNT TERRACE LAUNDRY BUILDING #4 | 2264 | 1979 | HOUSING | REVENUE | Building |
| W0L5 | YES | WYMOUNT TERRACE LAUNDRY BUILDING #5 | 2246 | 1985 | HOUSING | REVENUE | Building |
| W0MP | YES | WYMOUNT TERRACE MULTI-PURPOSE CENTER | 10635 | 1980 | HOUSING | REVENUE | Building |
| W10A | YES | WYMOUNT TERRACE (W10A) | 9436 | 1979 | HOUSING | REVENUE | Building |
| W10B | YES | WYMOUNT TERRACE (W10B) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W10C | YES | WYMOUNT TERRACE (W10C) | 9436 | 1979 | HOUSING | REVENUE | Building |
| W10D | YES | WYMOUNT TERRACE (W10D) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W10E | YES | WYMOUNT TERRACE (W10E) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W10F | YES | WYMOUNT TERRACE (W10F) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W11A | YES | WYMOUNT TERRACE (W11A) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W11B | YES | WYMOUNT TERRACE (W11B) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W11C | YES | WYMOUNT TERRACE (W11C) | 9436 | 1979 | HOUSING | REVENUE | Building |
| W11D | YES | WYMOUNT TERRACE (W11D) | 6320 | 1979 | HOUSING | REVENUE | Building |
| W11E | YES | WYMOUNT TERRACE (W11E) | 9436 | 1979 | HOUSING | REVENUE | Building |
| W12A | YES | WYMOUNT TERRACE (W12A) | 6320 | 1982 | HOUSING | REVENUE | Building |
| W12B | YES | WYMOUNT TERRACE (W12B) | 6320 | 1982 | HOUSING | REVENUE | Building |
| W12C | YES | WYMOUNT TERRACE (W12C) | 9436 | 1982 | HOUSING | REVENUE | Building |
| W12D | YES | WYMOUNT TERRACE (W12D) | 6320 | 1982 | HOUSING | REVENUE | Building |
| W13A | YES | WYMOUNT TERRACE (W13A) | 9436 | 1982 | HOUSING | REVENUE | Building |
| W13B | YES | WYMOUNT TERRACE (W13B) | 6320 | 1982 | HOUSING | REVENUE | Building |
| W13C | YES | WYMOUNT TERRACE (W13C) | 9436 | 1982 | HOUSING | REVENUE | Building |
| W13D | YES | WYMOUNT TERRACE (W13D) | 9436 | 1982 | HOUSING | REVENUE | Building |
| W14A | YES | WYMOUNT TERRACE (W14A) | 3257 | 1985 | HOUSING | REVENUE | Building |
| W14B | YES | WYMOUNT TERRACE (W14B) | 6514 | 1985 | HOUSING | REVENUE | Building |
| W14C | YES | WYMOUNT TERRACE (W14C) | 6514 | 1985 | HOUSING | REVENUE | Building |
| W14D | YES | WYMOUNT TERRACE (W14D) | 6514 | 1985 | HOUSING | REVENUE | Building |
| W15A | YES | WYMOUNT TERRACE (W15A) | 6514 | 1985 | HOUSING | REVENUE | Building |
| W15B | YES | WYMOUNT TERRACE (W15B) | 6514 | 1985 | HOUSING | REVENUE | Building |
| W15C | YES | WYMOUNT TERRACE (W15C) | 9728 | 1985 | HOUSING | REVENUE | Building |
| W15D | YES | WYMOUNT TERRACE (W15D) | 9728 | 1985 | HOUSING | REVENUE | Building |
| W15E | YES | WYMOUNT TERRACE (W15E) | 6514 | 1985 | HOUSING | REVENUE | Building |
| W16A | YES | WYMOUNT TERRACE (W16A) | 6514 | 1992 | HOUSING | REVENUE | Building |
| W16B | YES | WYMOUNT TERRACE (W16B) | 6514 | 1992 | HOUSING | REVENUE | Building |
| W16C | YES | WYMOUNT TERRACE (W16C) | 6514 | 1992 | HOUSING | REVENUE | Building |
| W16D | YES | WYMOUNT TERRACE (W16D) | 6514 | 1992 | HOUSING | REVENUE | Building |
| W16E | YES | WYMOUNT TERRACE (W16E) | 6514 | 1992 | HOUSING | REVENUE | Building |
| W16F | YES | WYMOUNT TERRACE (W16F) | 6514 | 1992 | HOUSING | REVENUE | Building |
| W16G | YES | WYMOUNT TERRACE (W16G) | 6514 | 1992 | HOUSING | REVENUE | Building |
| W17A | YES | WYMOUNT TERRACE (W17A) | 6514 | 1992 | HOUSING | REVENUE | Building |
| W17B | YES | WYMOUNT TERRACE (W17B) | 6514 | 1992 | HOUSING | REVENUE | Building |
| W17C | YES | WYMOUNT TERRACE (W17C) | 6514 | 1992 | HOUSING | REVENUE | Building |
| WCAS | YES | BYU WEST CAMPUS AUTO SHOP BUILDING | 16772 | 1980 | ACADEMIC | Investment Property | Building |
| WCCB | YES | BYU WEST CAMPUS CENTRAL BUILDING | 150556 | 1920 | ACADEMIC | Investment Property | Building |
| WCCL | YES | BYU WEST CAMPUS CLASSROOM BUILDING | 13690 | 1988 | ACADEMIC | Investment Property | Building |
| WCD | NO | BYU WEST CAMPUS D WING | 27308 | 1956 | ACADEMIC | Investment Property | Discontinued |
| WCDE | YES | BYU WEST CAMPUS DRIVERS EDUCATION BUILDI | 116 | 1997 | ACADEMIC | Investment Property | Building |
| WCF | YES | BYU WEST CAMPUS FIELD | 0 | 2018 | ADMIN | Investment Property | Facility |
| WCGB | YES | BYU WEST CAMPUS CENTRAL GYMNASIUM BUILDI | 39945 | 1920 | ACADEMIC | Investment Property | Building |
| WCGG | YES | BYU WEST CAMPUS GROUNDS GARAGES | 1200 | 1980 | ACADEMIC | Investment Property | Building |
| WCP1 | NO | BYU WEST CAMPUS PORTABLE P-1, P-2 | 1770 | 1988 | ACADEMIC | Investment Property | Discontinued |
| WCP3 | NO | BYU WEST CAMPUS PORTABLE P-3, P-4 | 1768 | 1988 | ACADEMIC | Investment Property | Discontinued |
| WCPB | YES | BYU WEST CAMPUS WEST PRESS BOX | 313 | 1980 | ACADEMIC | Investment Property | Building |
| WCPD | NO | BYU WEST CAMPUS DANCE PORTABLE | 1772 | 2008 | ACADEMIC | Investment Property | Discontinued |
| WCSB | YES | BYU WEST CAMPUS CENTRAL SOUTH BUILDING | 41839 | 1920 | ACADEMIC | Investment Property | Building |
| WCST | YES | BYU WEST CAMPUS STADIUM AND TRACK | 0 | 2018 | ADMIN | Investment Property | Facility |
| WCTC | YES | BYU WEST CAMPUS TENNIS COURTS | 0 | 2018 | ADMIN | Investment Property | Facility |
| WCWS | YES | BYU WEST CAMPUS WEST STAND | 5312 | 2013 | ACADEMIC | Investment Property | Building |
| WP | YES | WYVIEW PARK GENERAL |  |  |  | BILLING | General Site |
| WP01 | YES | WYVIEW PARK (WP01) | 15606 | 1998 | HOUSING | REVENUE | Building |
| WP02 | YES | WYVIEW PARK (WP02) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP03 | YES | WYVIEW PARK (WP03) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP04 | YES | WYVIEW PARK (WP04) | 15629 | 1998 | HOUSING | REVENUE | Building |
| WP05 | YES | WYVIEW PARK (WP05) | 15629 | 1998 | HOUSING | REVENUE | Building |
| WP06 | YES | WYVIEW PARK (WP06) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP07 | YES | WYVIEW PARK (WP07) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP08 | YES | WYVIEW PARK (WP08) | 15609 | 1998 | HOUSING | REVENUE | Building |
| WP09 | YES | WYVIEW PARK (WP09) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP10 | YES | WYVIEW PARK (WP10) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP11 | YES | WYVIEW PARK (WP11) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP12 | YES | WYVIEW PARK (WP12) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP13 | YES | WYVIEW PARK (WP13) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP14 | YES | WYVIEW PARK (WP14) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP15 | YES | WYVIEW PARK (WP15) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP16 | YES | WYVIEW PARK (WP16) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP17 | YES | WYVIEW PARK (WP17) | 15608 | 1998 | HOUSING | REVENUE | Building |
| WP18 | YES | WYVIEW PARK (WP18) | 15625 | 1998 | HOUSING | REVENUE | Building |
| WP19 | YES | WYVIEW PARK (WP19) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP20 | YES | WYVIEW PARK (WP20) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP21 | YES | WYVIEW PARK (WP21) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP22 | YES | WYVIEW PARK (WP22) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP23 | YES | WYVIEW PARK (WP23) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP24 | YES | WYVIEW PARK (WP24) | 15595 | 1998 | HOUSING | REVENUE | Building |
| WP25 | YES | WYVIEW PARK (WP25) | 10653 | 1998 | HOUSING | REVENUE | Building |
| WP26 | YES | WYVIEW PARK (WP26) | 15628 | 1998 | HOUSING | REVENUE | Building |
| WP27 | YES | WYVIEW PARK (WP27) | 15608 | 1998 | HOUSING | REVENUE | Building |
| WP28 | YES | WYVIEW PARK (WP28) | 15608 | 1998 | HOUSING | REVENUE | Building |
| WP29 | YES | WYVIEW PARK (WP29) | 10652 | 1998 | HOUSING | REVENUE | Building |
| WP30 | YES | WYVIEW PARK (WP30) | 15608 | 1998 | HOUSING | REVENUE | Building |
| WPCB | YES | WYVIEW PARK CENTRAL BUILDING | 3886 | 1998 | HOUSING | REVENUE | Building |
| WPMB | YES | WYVIEW PARK MAINTENANCE BUILDING | 1140 | 1997 | HOUSING | REVENUE | Building |
| WPMP | YES | WYVIEW PARK MULTI-PURPOSE BLDG | 12330 | 1998 | HOUSING | REVENUE | Building |
| WPRA | YES | WYVIEW PARK RECREATION AREA |  | 1998 | HOUSING | APPROPRIATED | Facility |
| WSC | YES | WILKINSON (ERNEST L) STUDENT CENTER | 497574 | 1964 | AUXILIARY | REVENUE | Building |
| WT | YES | WYMOUNT TERRACE GENERAL |  |  |  | BILLING | General Site |
| WTHR | YES | WEATHER STATION | 42 | 1980 | ACADEMIC | APPROPRIATED | Building |
| WTRA | YES | WYMOUNT TERRACE RECREATION AREA |  | 1963 | HOUSING | REVENUE | Facility |
| WVB | YES | WEST VIEW BUILDING |  | 2020 | ACADEMIC | APPROPRIATED | Building |
| YMRR | YES | Y MOUNTAIN RESTROOM | 485 | 2016 | ADMIN | APPROPRIATED | Building |
| YMTN | YES | Y MOUNTAIN | 0 | 1906 | ADMIN | APPROPRIATED | Facility |
| ZNOB | YES | NO BUILDING |  |  |  | BILLING | General Site |

<C lick to add a one-page resume for each team member with each resume on a new page by itself.  Add other Appendices as necessary for data, numerical results and result summary tables, software/app source code & sample software/app execution, etc.>