# Mitigating submerged hydraulic jumps at low-head dams

by Team 11 | Dam solutions

Jason Poff
Emily Dicataldo
Patrick McDivitt
andrew meyer

A Capstone Statement of Work

submitted to
dr. rollin hotchkiss
brigham Young University

Department of Civil and Construction Engineering
Brigham Young UniverSity
November 22nd 2021

# Introduction

# PROJECT TITLE:	Mitigating SUBMERGED hydraulic jumps at low-head dams

# PROJECT ID:		CCE_CPST_011

# PROJECT SPONSOR:	Dr. Rollin Hotchkiss

# TEAM NAME:		Dam Solutions

This statement of work will provide an outline of the project objectives, tasks to be completed, proposed schedule, and expected deliverables. This project is an attempt to solve the dangers surrounding low-head dams. Low-head dams are short structures, typically less than 15 feet tall, extending from bank to bank across a river. These dams are typically built to divert water for irrigation or other purposes. Under high flow conditions, a phenomenon known as a submerged hydraulic jump can occur. Fast moving water falls over the crest of the dam and meets deep slow-moving water (high tail-water). The fast-moving water rises and is pushed back to the face of the dam, creating a recirculating current. These currents are extremely difficult to escape, causing those caught in them to be trapped and drown. Our task is to create a scaled modification of the low-head dam in the Jordan river by W 4430 S and S 500 W in Murray, Utah, a known location of a submerged hydraulic jump. This modification needs to prevent the formation of the submerged hydraulic jump and thereby prevent the dangerous current. The design must work for varying levels of flows in the Jordan river, be relatively inexpensive, and not create new hazards. The model will be scaled for and tested in the BYU flume.

Our end goal is to produce three suitable retrofit designs, one working model of our best design, and a comprehensive final report and presentation summarizing our work. We will accomplish this by reviewing literature about previous work done relating to our project, surveying the low-head dam located in Murray, Utah, creating a scaled model of the stream bed, designing three retrofitted models, building and testing our final design, and then creating our final report and presentation. This project will take place over the course of 5 months and will conclude at the end of March 2022.

# Proposed Work Plan

1. Survey the stream bed of the low-head dam by W 4430 S and S 500 W in Murray, Utah: In November with BYU Risk management, use a surveying rod and a volunteer in a wetsuit to gather points along the stream bed by the dam in Murray. Make sure the volunteer is tied off safely and other appropriate safety measures are taken. Take the points along the base of the dam, and at regular intervals away from the dam across the width. Take points up to 100 feet downstream of the dam.

2. Create a scaled model form of the streambed to be used in the flume at BYU: In December, use interpolation between points to create a 3D model of the streambed. Make sure this model is scaled to fit the flume in the lab. Give those points to the manufacturing lab and have a foam model made. Dr. Hotchkiss will make sure this lab still exists, and if not will instruct us how to construct the model ourselves.

3. Do a literature review of modifications to low-head dams: In November and December, conduct a literature review of low-head dam modifications. Use Patrick and Jason’s past literature reviews on low-head dams as a start to collect ideas for dam modification. Find any more strategies we missed and compile them into a document.

4. Discuss goals of the remodel and create measures to judge designs by: In January, meet with Dr. Hotchkiss and go over desired outcomes of the dam modification. Develop ranking mechanisms to judge designs by. Clearly define these in a point system.

5. Design 3 remodel options: In January/February, use the literature review to design several remodel options on paper. Choose designs we feel would score the best according to our point system. Find the range of flows that come through the Jordan river through online resources (USGS), and design for the 50-year flood flow.

6. Rank the designs by the measures we created: In January/February, rank each design together as a group. Use the point system we created earlier and choose the option that scores the best.

7. Build a model of the best solution: In February, create a scaled 3D model of the solution in AutoCAD that fits the flume. Submit it to the manufacturing team if they still exist. They will use our information to build a foam model for us that we can use. If they do not, Dr. Hotchkiss will instruct us how to build the model ourselves.

8. Test the model in the flume: In March, put the model in the flume in a submerged hydraulic jump condition and see if the jump is disrupted. A scaled model of dam modification that successfully prevents the formation of a hydraulic jump in the flume is our exit criteria. If it does not disrupt the jump, tweak the design, and seek advice from Dr. Hotchkiss. Resubmit the design to be made and try again.

9. Prepare a report of the work done: In March, compile all the work into a report and poster according to the requirements defined below in the deliverables. Completion and submittal of this report to our sponsor indicates we have finished our work.

# Schedule

Teams will meet as a group for a minimum of 30 minutes every week until the end of capstone, meetings will take place Mondays at 5pm or scheduled as necessary. Meetings with sponsors will take place twice a month to ensure that the project is being completed to the standards of the customer.

November 2021- Once Dam Solutions has received approval from BYU risk management, team will travel to the project site and survey the low-head dam crossing on the Jordan River in Murray.

December 2021- Use data collected while surveying to create a 3-D model of the streambed and give data to manufacturing lab to create model for BYU’s laboratory flume.

January 2022- With guidance from the team’s faculty advisor and relevant research, discuss retrofit option ideas applicable to low-head dam by W 4430 S and S 500 W in Murray, Utah.

February 1st, 2022- Produce 3 potential retrofit options and rank them as a group.

February 28th, 2022- Decide on one potential retrofit design and submit it to the manufacturing team to design.

March 2022- Tested the designed model in the laboratory flume and determine its ability to prevent the formation of a hydraulic jump.

April 8th, 2022- Team members will present a report to the client including design drawings, experimental results, cost estimates of the prototype, as well as impacts this adjustment might have on society.

# Facilities, Tools, Data and Equipment

We plan on designing our model of the low head dam by surveying the dam, the stream bed immediately downstream from the dam, and the stream bed ~100 ft downstream of the dam. We will accomplish this by having a student hold a surveying road in the river long enough to get readings at each location. This student will be equipped with a wetsuit and helmet and will also be tied off with two ropes. Two people on each side of the river will hold on to these ropes. We plan on hiring a machine shop to create a scaled 3D model of the surface of the stream bed made of foam. Using the data obtained from surveying, we will design three retrofitted dams using a 3D modeling program. Then using the laboratory flume in CB 171, we will install and test the 3D foam surface and our final design.

# Project Budget

| Timeline | Tasks | Time Needed to Complete Task |
| --- | --- | --- |
| November 2021 | Literature review | 100 Hours |
| TBD – December 2021 | Surveying Excursion | 8 Hours |
| December 2021 | Create scaled model of stream bed in 3D modeling program from surveying data | 100 Hours |
| January and February 2022 | Design 3 retrofitted models and determine best one | 200 Hours |
| March 2022 | Build, install, and test final design | 200 Hours |
| March 2022 | Create final report | 200 Hours |
|  | TOTAL: | 808 Hours |

# Deliverables

The end deliverable product will be a plan set depicting the most suitable retrofit options on the XX low head dam located in Murray, Utah.

- Monthly status reports:

  - Answer the following questions:

    - What challenges have we faced in our Capstone project?

    - What actions did we take to overcome these challenges?

    - What progress did we make in overcoming these challenges?

    - Is the project on schedule?

  - Summarize the progress and status of our Capstone project

  - In what ways did the challenges negatively affect our project? What are we doing to get back on schedule?

- Scaled models with suitable retrofit options

- Final report with suitable retrofit options which includes economic and environmental impacts

- Poster that summarizes our project in a presentable manner for students, faculty, and other interested individuals

- Presentation to be presented to Dr. Hotchkiss

Other deliverables agreed upon with Dr. Hotchkiss

# Performance Standards

Team will provide work for this Capstone project “as is” using best practices and with best effort. Project results cannot be construed as work performed by licensed professionals and cannot be used as “stamped deliverables” without first being reviewed, approved, and stamped by a qualified and relevant licensed professional engineer.

The team is to create a working model of a low-head dam, modeled after a high-risk zone on the Jordan River in Murray Utah, that mitigates the formation of a recirculating current. The prototype should additionally reduce hazards of the low-head dam for a variety of discharges and tailwater elevations. By the end of the project, team members will present design drawings, experimental results, cost estimates of the prototype, as well as impacts this adjustment might have on society.

# Statement of Qualification

Emily Dicataldo:

- Senior in Civil Engineering at BYU

- 8 months of low-head dam inventory on Google Earth

- Relevant courses: Fluid Mechanics, Hydraulics, and Hydrology at BYU

- Internship at Hansen, Allen, and Luce (water engineering consulting firm)

Jason Poff:

- Senior in Civil Engineering at BYU

- Passed the FE

- Wrote literature review on low-head dams

- 9 months of low-head dam inventory on Google Earth

- Researched 16 court cases involving deaths at low-head dams

- Presented poster about low-head dams at the AWWA conference

- Relevant courses: Fluid Mechanics, Hydrology, GIS

- Phi Kappa Phi Honor Society Invitation

- Golden Key Honor Society Invitation

Andrew Meyer:

- Senior in Civil and Construction Engineering at BYU

- Completed CE 332: Fluid Mechanics

- GeoStrata Laboratory Technician

Patrick McDivitt:

- Senior in Civil Engineering at BYU

- Passed the FE

- Taken CE 332: Fluid Mechanics

- AECOM Construction Services Intern (2 Years)

- BYU Phi Eta Sigma member for exceptional grades

Organizational Structure of Team:

- Jason Poff:

  - Team Captain

  - Main contact point for Dr. Hotchkiss and third-party contacts

  - Identifies objectives, plans, and organizes team to achieve objectives

- Emily Dicataldo:

  - Drafting Lead

  - In charge of drafting plans detailing retrofit options (for both prototype and model scales)

- Andrew Meyer:

  - Flume Lead

  - In charge of reserving, running, and using the flume to analyze various scaled retrofit options

- Patrick McDivitt:

  - Results Lead

  - Will use fluid mechanics concepts to interpret results and the effectiveness of various retrofit options

# Appendix A